Technical Reference

Sutron Corporation

21300 Ridgetop Circle

Sterling, Virginia 20166

TEL: (703) 406-2800

FAX: (703) 406-2801

WEB: http://www.sutron.com/

Table Of Contents

Welcome to the Technical Reference	1

DDE Concepts	3

XConnect and OPC	5

OPC Version	5

OPC Clients tested with XConnect OPC Servers	6

OPC Test Software	7

XConnect Implementation	7

Example 1	7

Example 2	7

Example 3	8

Example 4	8

Example 5	8

Additional References	8

XC Setup	9

XC Setup	9

Introduction	9

DDE Information	9

Setup Information	10

XC Setup COM Interface	13

Introduction	13

XCSetupCOM Methods	13

XCSetupCOM Methods	15

Methods	15

Events	40

XC Rtu	43

XC Rtu DDE Interface	43

Introduction	43

DDE Information	43

Setup Information	58

XC Rtu COM Interface	74

Introduction	74

XCRtuDataCOM Methods	74

XCRtuDataCOM Events	74

Methods	75

Events	76

XC Mux	79

XC Mux DDE Interface	79

Introduction	79

DDE Information	79

Setup Information	81

XC Mux COM Interface	83

Introduction	83

XC Decode	85

XC Decode DDE Interface	85

Introduction	85

DDE Information	85

Setup Information	88

XC Decode COM Interface	95

Introduction	95

XCDecodeCOM Methods	95

XCDecodeCOM Events	95

Methods	95

Events	97

XC Alarm	99

XC Alarm COM Interface	99

Introduction	99

XCAlarmDataCOM Methods	99

XCAlarmDataCOM Events	99

Methods	99

Events	101

XC Calc	103

XC Calc COM Interface	103

Templates	105

NESDIS Header Field Description	107

# Welcome to the Technical Reference

This document describes all the setup fields, and DDE/COM commands supported by the XConnect applications. This information can be used to build add-on products for XConnect, interface to existing XConnect servers with 3rd party software, or design custom setups and applications for situations beyond the scope of the standard setup and operator programs. Knowledge of the fields and setup information can used and tested with applications ranging from the very simple Microsoft Excel, Microsoft Word, Visual Basic, Intouch up to high level languages such as C and Pascal. The sections are organized by DDE information and Setup information. In general the DDE information is used to control and access an applications in real-time, whereas the setup information is used to define the default behaviour and operation of the application. A brief introduction is included to each application, and meant to be of a more technical nature than the information contained in the individual XConnect applications' help file.

This document describes an open, expandable and flexible standard for data collection and storage, which can grow as technology and requirements develop.

# DDE Concepts

DDE stands for Dynamic Data Exchange and is a Windows based messaging protocol for sharing information between applications.  DDE is the basis for all information exchange and commands in the PC Base 2 software.  Much of the DDE interaction between applications is automatic and hidden from the user, but since DDE is supported by many non-Sutron applications it is very useful to understand the basics of DDE so that these applications can interact with the base station software.

A DDE message consists of four parts:

- the name of the application receiving the message

- the topic which the message is about

- an item under that topic

- data

The application name is typically limited to 8 characters, such as XCSETUP, XCRTU, or EXCEL.  If NETDDE is installed, then the application name can also contain a network node name, such as \\WORKSTATION\XCSETUP would address the XC Setup running on network node WORKSTATION. The topic name will depend on the application, most of the topics are named DATA or CONTROL in XConnect.  DATA topics usually supply or change information, whereas CONTROL topics usually initiate or check the status of actions.  The data itself can be transferred in any format which the sending and receiving applications can agree upon.  The most basic formats are called TEXT for tab delimited ASCII text and CSV for comma delimited ASCII text.  Most applications support at least the TEXT format.  These formats are also the same formats as used by the Windows clipboard, hence they are often called clipboard formats.

DDE supports seven basic commands:

- INITIATE -- is used to start a DDE conversation between two applications and requires an application name and a topic name to do so.

- TERMINATE -- is used to end a DDE conversation on a particular topic.

- ADVISE -- is used to request a hot or automatic link to a data item, so that whenever the data item changes the applications which requested the advise link will be updated automatically.

- UNADVISE -- terminates an advise link.

- REQUEST -- is used to request the current value of a data item.

- POKE -- is used to set the current value of a data item.

- EXECUTE -- is used to execute a macro or list of commands specified by the data item.

NOTE: The syntax and format of commands, data, and responses is not standardized in DDE and hence this online help exists to describe the formats adopted by Sutron.

Sutron software uses the same notation for DDE paths as used by Microsoft EXCEL: app|top!item to specify a DDE item or just app|top to specify a DDE topic where the character "|" vertical bar separates applications from topics and the character "!" exclamation point separates topics from items.  Specifying a network source for a DDE path is performed by placing \\node\ before the application.

# XConnect and OPC

The XConnect servers are automation servers built on Microsoft Windows COM technology.  As a natural extension, XConnect has implemented OPC (OLE for Process Control) and XConnect applications such as XC Rtu, XC Decode are OPC server.  Like DDE, OPC allows applications to share data via standard interface.  In this case, OPC standardizes the communication for process control data.  OPC is a communication standard.

When data acquisition devices conform to the OPC standard, you can use them with any OPC-enabled software application. And vice versa. You can therefore easily combine different devices from different manufacturers in one system, without the normal integration headaches. OPC gives you the freedom to add new hardware from third-party vendors to existing set-ups, or to replace a device, without worrying about compatibility with your chosen software. Select exactly the hardware and software you want for a particular application.

Based on Microsoft Windows technology, OPC now stands for Open Process Control. Previously it stood for OLE Process Control but today the OLE technology has been replaced by Active X.

The measurement and control hardware provides front-line data acquisition. As soon as the hardware device has collected the data it makes it available to software applications running under Windows. It presents the data according to the OPC standard, and is thus known as an OPC server.  Each OPC server offers data in the same way. If the software application can understand the OPC format it can therefore access data from any OPC server device, making individual drivers for each piece of equipment obsolete.

OPC-enabled software include spreadsheets, databases, virtual instruments and SCADA (supervisory control and data acquisition) interfaces. These applications are known as OPC client software. You can also develop your own Visual Basic programs to exchange data with OPC servers. Your program will work with all OPC devices connected to your PC and on the network.

Each OPC server can simultaneously provide data for any number of OPC clients. Likewise multiple clients can at the same moment access any server: a robust method of communication. With OPC, measurement and control systems can share information and co-operate with other installations across factories, offices, laboratories, etc. The same data is therefore readily available to engineering, maintenance, management...in fact to anyone that requires up-to-the-minute data on which to base their decisions.

OPC allows "plug-and-play". All OPC devices will connect together and immediately work with the OPC client software. This has the potential to massively reduce installation and system configuration time. It also means that you can add devices without shutting down existing systems.

## OPC Version

XConnect has implemented OPC Data Access Custom Interface Version 2. The details on interfaces implemented are listed below:

- OPC Server

  - IOPCServer

  - IOPCServerPublicGroups

  - IOPCBrowseServerAddressSpace

  - IOPCItemProperties

  - IConnectionPointContainer

  - IOPCCommon

- OPC Group

  - IOPCGroupStateMgt

  - IOPCPublicGroupStateMgt

  - IOPCASyncIO2

  - IOPCItemMgt

  - ICOnnectionPointContainer

  - IOPCSyncIO

- EnumOPCItemAttributes

  - IEnumOPCItemAttributes

## OPC Clients tested with XConnect OPC Servers

- Wonderware Intouch 8.xx/OPC Link 7.6.1.9 in compliance with OPC Data Access version 2.04.

- Matrikon OPC Explorer (http://www.matrikonopc.com/products/opc-desktop-tools/opc-explorer.asp)

## OPC Test Software

- Wonderware Intouch / OPC Link

- OPC Foundation Compliance Test Utility

## XConnect Implementation

In general, the XConnect servers try to preserve the same syntax as DDE tag names.  They follow the DDE data points as described in XConnect Tech Note.  The DDE topic name is the first access path followed by a dot “.” and then the item name.

- Because all DDE points can be read/write, all OPC tagnames are writable as well.

- All values returned are variants, so that they can be easily converted to other data types just as DDE.

- If the values for tagnames are updated, the clients are notified only when the values are changed.  No notifications are sent out if the update is the exact same value as the old one.

### Example 1

DDE - Application - XCRtu; Topic - Data; Item - XPERT.AT.

OPC - The sensor AT on station XPERT can be accessed as DATA.XPERT.AT.

### Example 2

DDE - Application - XCRtu; Topic - Data; Data - $CURRENT.

OPC - The current string can be accessed as DATA.$CURRENT.

### Example 3

DDE - Application - XCRtu; Topic - Control; Item - POLL,XPERT,CURRENT; Data - 10.

OPC - Set CONTROL.POLL,XPERT,CURRENT to value 10 will trigger the current poll to station XPERT.

### Example 4

DDE - Application - XCRtu; Topic - Control; Item - POLL,XPERT,DATE; Data - 10.

OPC - To poll the station using Date poll, write/set the item CONTROL.POLL,XPERT,DATE to 10.

### Example 5

OPC - Special item CONTROL.EXECUTE is available for other control functions.  Set this item the value in the syntax as “key = value”. Set CONTROL.EXECUTE to string POLL,XPERT,CURRENT=10 will trigger the current poll to station XPERT.  This is a generic item that can be used to create additional tagnames in the OPC server address space.

## Additional References

For more information, view documents listed below at http://www.opcfoundation.org.

- OPC Overview 1.0

- OPC Common Definitions and Interfaces 1.0

- OPC Data Access Custom Interface Standard 2.05

# XC Setup

## XC Setup

### Introduction

The XC Setup is designed to isolate XConnect applications from the details of how setup information is stored and accessed.  This allows the setup information for any application to be modified locally or remotely by any other application using DDE/NETDDE.  The current implementation of the XC Setup stores setup information in an INI file and a database.  The information in the file XCONNECT.INI is an ASCII format, similar to other Windows .INI files.  Application and data storage setup information is contained in the INI file.  Station, sensor and pollgroup setup information is stored in a database.  Because applications often cache setup information in their own local memory, the XC Setup supports a method of notifying applications that setup information has been changed.

Changing the default file is useful if you need to maintain multiple setups.  From the XC Setup main menu, select File|Open and select the desired filename.

The second way is to pass the filename on the command line to XC Setup. Since XC Setup is automatically started by XConnect server programs with an empty command line, you will have to make sure that you start XC Setup with the correct command line parameter before running other XConnect programs. You can select Options|Make INI Default from the XC Setup main menu to ensure the new filename is used as the default name (instead of XCONNECT.INI) when XC Setup is started

Warning: Although you can edit the XCONNECT.INI directly with the notepad, edit or any ASCII text file editor, do not do so while the XC Setup is running - shut it down first!  You can make changes to the setup while the base station is running using DDE, but be aware that many applications cache the setup information and will not notice the changes you have made unless they are told to reset their caches or restart.  See the discussion about "GENERAL,RESETSETUP".

Data is stored in the XCONNECT.INI files under a topic and an item, the general format is show below:

[topic]

item=some kind of ASCII printable data

There can be an unlimited number of topics in a setup and an unlimited number of items under any topic.  Data must be less than 64K bytes, and must consist of characters between space (32) and tilde (127) in the ASCII sequence.  There is one DDE topic under XC Setup.

### DDE Information

DDE APPLICATION NAME:   XCSETUP

DDE TOPIC NAME:   DATA

| ITEM | A | P | R | E | DATA | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
| topic,item | x | x | x |  | any string | Data in the XC Setup is stored under a topic and an item name, a topic and an item can be assigned any text value. Operation will fail if the data does not exist in the setup. |
| topic,* |  |  | x |  | topic list | When a "*" is specified for the item, a list of all items under the specified topic will be returned. |
| +topic,item |  | x |  |  | any string | Poking with a "+" sign before the topic will force the item to be added to the setup if it does not already exist. |
| +topic,item |  |  | x |  | +¦-any string | Requesting with a "+" sign before the topic will cause a “+” sign to be inserted before the returned data if the data was accessed thru a template or a “-” sign if it was not. |
| -topic,item |  |  | x |  | any string | Requesting with a "-" before the topic sign will disable template searching. |
| ADD topic,item |  |  |  | x | n/a | Adds a topic and an item to the setup initializing it to blank. |
| DEL topic,item |  |  |  | x | n/a | Deletes an topic and an item from the setup. |
| UPDATE |  |  |  | x | n/a | Causes the XC Setup to write all changes in memory out to disk. |

Operations: A-Advise/UnAdvise, P-Poke, R-Request, E-Execute

### Setup Information

XC Setup does not use much setup information, its job is to supply setup information to other applications.  One piece of information which all applications which use the XC Setup share in common via an advise link is the "GENERAL,RESETSETUP" item which when poked with the values below should cause the following actions:

1: Minor setup changes to status information such as station/sensor setups has occured, All applications should flush their setup information caches.

2: Major setup changes to configuration information such as COM ports and data files has occured, all applications should close active functions which rely on this information, re-read setup, and restart.

3: Reset DDE communications, useful when automatic links may have been terminated and/or re-started and applications are no longer in sync.

9: XC Setup has shutdown, and all users of setup information should also shutdown.

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [SETUPSERVER] | UpdateInterval | 600 | UpdateInterval contains the number seconds between disk writes performed by the XC Setup. If another value is not specified the XC Setup will write changed setup information out to disk every 10 minutes. If no data has been changed then the XC Setup does not write to disk. A long interval (such as 10 minutes) will reduce disk writing and help keep the system running faster (updating a Windows .INI file is a very disk intensive). The risk of a long interval is that updates might be lost if power is lost or the system crashes between updates. Client applications should instruct XC Setup to update after making significant changes to the setup. |

The [GENERAL] topic contained in the following table contains setup items shared by all XConnect server applications:

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [GENERAL] | Version | 2 | Version number of setup file, as new features are introduced this number may be increased. |
|  | DataPath |  | Default directory path to data files. |
|  | SetupPath |  | Default directory path to setup files. |
|  | TablePath |  | Default directory path to setup files. |
|  | LogPath |  | Default directory path to store Logger’s log files. |
|  | RawPath |  | Default directory path to store raw (non-decoded) NESDIS message files. |
|  | Use24Hour | No | When set to "Yes" midnite will be displayed as 24:00 hours, when set to "No" midnite will be displayed as 00:00 hours. This option does not effect how the data is stored. |
|  | ResetSetup |  | see table above. |
|  | INIFilename | XCONNECT.ini | Contains the file name of the setup file currently in use. The value is (read only) poking it will not change the setup file. To change the default setup file assign the filename to the DOS environment variable "XCONNECT.INI" before running windows, or pass the filename on the command line to XC Setup before starting other elements of XCONNECT. The default ini file can be changed from XC Setup. The name of the file will also be stored in the registry. |
|  | GMT | 0 | Number of hours Greenwich Mean Time (GMT) is ahead of Local Time. Currently used by the GOES XC Mux program to time stamping NESDIS compatible files with GMT time. Also used by the GOES XC Decode program in order to extract the time stamp from the same files. If you only ever going to use NESDIS files retrieved from NESDIS or from users in other time zones, then it is important to enter the correct GMT offset for your time zone. Use a negative value to indicate the Greenwich Mean Time is behind Local Time. |
|  | Stations |  | List of defined stations. ex: PumpPlant,MetSite,StageSite |
|  | DecodeAll | No | Set to Yes will causes the XC Decode to at least partially decode all messages regardless of whether the SatID of the message is defined. This is useful when debugging because the decoder can decode the data quality information without needing setup information, but is undesirable in a final system because it causes the decoder to perform more processing than is necessary. |
|  | UseNoValue | No | If Yes, then a value decode by XC Rtu or XC Decode as nil or missing will be replaced with the NoValueIndicator. |
|  | NoValueIndicator | 998877.0 | If UseNoValue is Yes, the value for NoValueIndicator will be used for any value decode as missing or nil. |

## XC Setup COM Interface

### Introduction

The XC Setup is designed to isolate XConnect applications from the details of how setup information is stored and accessed.  This allows the setup information for any application to be modified locally or remotely by any other application using COM/DCOM.  The current implementation of the XC Setup stores setup information in an INI file and a database.  The information in the file XCONNECT.INI is an ASCII format, similar to other Windows .INI files.  Application and data storage setup information is contained in the INI file.  Station, sensor and pollgroup setup information is stored in a database.  Because applications often cache setup information in their own local memory, the XC Setup supports a method of notifying applications that setup information has been changed.

Changing the default file is useful if you need to maintain multiple setups.  From the XC Setup main menu, select File|Open and select the desired filename.

XC Setup supports a variety of methods and events through its interface.  The XC Setup interface is named XCSetupCOM.  The methods and events are described below.  Most methods pass a group of parameters, a record, as a variant.  The order of the parameters in the table indicate is the order of the variant record that is passed.  Others functions will pass values as strings

| Type | Field Size |
| --- | --- |
| Boolean | 1 byte |
| Word | 2 bytes (unsigned) |
| Integer | 4 bytes (signed) |
| Longint | 4 bytes (signed) |
| Enumerated | 1 byte |
| Single | 4 bytes |
| Double | 8 bytes |

### XCSetupCOM Methods

- GeneralSetup

- RtuServerSetup

- ComSetup

- StationSetup

- SensorSetup

- SensorDecodeSetup

- DecoderSetup

- MuxSetup

- DAPSSetup

- PollGroupSetup

- DataServerSetup

- DataStoreGroupSetup

- DeleteTopicItem

- AddTopicItem

- SingleItem

- ResetAllParams

- AppShutdown

- SetItemLink

- StationListDetails

- RtuPortListDetails

- MuxPortListDetails

- PollGroupListDetails

- DataStoreGroupListDetails

- DeleteStation

- DeleteSensor

- DeleteComPort

- DeletePollGroup

- DeleteDataStoreGroup

- AddStation

- AddSensor

- AddComPort

- AddPollGroup

- AddDataStoreGroup

- ResetMuxParms

- ResetDecoderParms

- ResetRtuServerParms

- ResetSensorDecodeParms

- ResetDAPsParms

- LoadSetup

- AssignTemplate

- UnAssignTemplate

- FromTemplate

- RenameComPort

- SaveINIFile

- SetItemChange

- UserLoginChange

- IsAlive

- CurrentUser

- ResetDBParms

- ResetDBPollGroups

### XCSetupCOM Methods

- OnResetParams

- OnResetAllParams

- OnAppShutdown

- OnResetCOMParams

- OnItemChange

- OnMuxParamsChange

- OnDecoderParamsChange

- OnRtuServerParamsChange

- OnSensorDecodeParams

- OnDAPSParamsChange

- OnUserLoginChange

- OnHeartBeat

### Methods

#### GeneralSetup

This function allows the user to retrieve or set General Setup parameters.  This is a read-write method.  The GeneralSetup parameter record is defined below.  These represent the same fields as listed in the XCONNECT.INI file under  the [General] topic.  A variant array is passed in or returned based on the GeneralSetup parameter record order below.

##### GeneralSetup Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| Version | String | INI file version number. Current version is 3. | Blank string |
| DataPath | String | Directory path for PCBASE2 data files, if used for data storage. | Blank string |
| SetupPath | String | Directory for setup files. | Blank string |
| TablePath | String | Directory where rating table files (.TBL) are stored. | Blank string |
| LogPath | String | Directory where .LOG/.XML files will be stored. Used for LOGGER or XML data storage options | [XConnect Install Dir]\Log\ |
| RawPath | String | Directory where RAW files will be stored by XC Mux. | [XConnect Install Dir]\Raw\ |
| Stations | String | Comma-delimited string of station names. | Blank string |
| Use24Hour | Boolean | Obsolete field (seconds). | False |
| MaxDDEData | Word | Obsolete field (seconds). | 60000 |
| MaxAutoData | Word | Obsolete field (seconds). | 60000 |
| MaxDDETime | Integer | Obsolete field (seconds). | 180000 |
| MaxAutoTime | Integer | Obsolete field (seconds). | 2000 |
| GMTToLocal | Boolean | Use GMT offset based on Windows setting to adjust all incoming satellite message time stamps. | False |
| CustomGMT | Boolean | Flag indicates to use user-defined GMT offset instead of Windows GMT setting. | False |
| GMT | Integer | User-defined GMT offset. | 0 |
| DecodeAll | Boolean | Used in satellite decoding, decodes all satellite messages in a raw file. | False |
| UseRtuServer | Boolean | Flag indicates conventional system that will use XC Rtu. | False |
| UseDecoder | Boolean | Flag indicates satellite system that will use XC Decode. | False |
| UseMux | Boolean | Flag indicates satellite system that will use XC Mux. | False |
| UseDaps | Boolean | Flag indicates satellite systems that will use XC Daps. | False |
| UseDataStore | Boolean | Flag indicates data will be stored, using XC DataStore. | False |
| DefaultStations | String | Comma-delimited string of template station names. | Blank string |
| DefaultSensors | String | Comma-delimited string of template sensor names. | Blank string |
| UseNoValue | Boolean | Substitute another value respresented by the NoValueIndicator parameter when the logger transmits a value with bad quality. | False |
| NoValueIndicator | Integer | Number that will be substituted when a value returned from the data logger has bad quality. | 998877 |
| AllNoValue | Boolean | Pass the NoValueIndicator value for follow-on computations (i.e., XC Calc, XC Alarm). | False |
| IniFileName | String | Name of current INI filename loaded. | Blank string |
| SystemName | String | Should be computer name - used in PollgroupSetup to represent owner name. Not used for polling like Master ID. | Blank string |

#### RtuServerSetup

This function allows the user to retrieve or set XC Rtu Setup parameters.  The RtuServerSetup parameter record is defined below.  These represent the same fields as listed in the XCONNECT.INI file under the [RtuServer] topic.  A variant array is passed in or returned based on the RtuserverSetup parameter record order below.

##### RtuServerSetup Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| MasterID | String | Unit ID of XConnect system. Used for SSP communications. | XCONNECT |
| ListenID | String | Other Master IDs or Unit IDs that XC Rtu will monitor, receive and parse the SSP messages; otherwise will ignore. | Blank string |
| ComPort | String | Comma-delimited list of active com/ip ports (i.e., COM1,600). | Blank string |
| PollGroups | String | Comma-delimited list of poll groups created (i.e., 1,2,3). This string created after reading all the poll groups from database. | Blank string |
| Computers | String | Obsolete parameter. | Blank string |
| Run | String | Comma-delimited list of applications that will be started when XC Rtu starts (i.e., XCAlarm.exe,Excel.exe,Display.exe). | Blank string |
| DDETimeout | Integer | Obsolete field (seconds). | 10000 |
| NetDDETimeout | Integer | Obsolete field (seconds). | 60000 |
| AutoPoll | Integer | Determines how often (in seconds) GETTAG controls will be issued to RTUs in order to update hot-linked data points in the AUTO topic. A value of 0.0 disables automatic polling. A negative number will cause a single poll to take place using a delay based on the absolute value, and then AUTOPOLL will be reset to 0. | 0 |
| AutoUpdate | Integer | Determines how long the XC Rtu will wait before issuing SENDTAG control to update RTUs after a data point in the AUTO topic has been changed. A value of 0.0 disables automatic updating. A negative number will cause a single pol to take place using a delay based on the absolute value, and then AUTOUPDATE will be reset to 0. | 0 |
| PacketSize | Integer | Size (bytes) of an SSP message - max is 1024. | 1024 |
| RePollLimit | Integer | When XC Rtu polls for time tag data from a unit and the response contains decoded sensors which are empty then XC Rtu will only update the time of LastPoll if the time stamp is older than RePollLimit. This handles two conflicting situations ... 1) The RTU may still be collecting data which was empty and a repolling later will collect it, and 2) The entry may have been skipped and all the re-polling in the world may never retrieve a value. Since an RTU usually logs data in “real time” and rarely logs data back in time, RePollLimit should be set to at least the longest logging interval in the system. RePollLimit has a second use in conjunction with the TimeLimit option under [POLLGROUP:n]. When a TimeLimited poll receives no data, the LastPoll date will be automatically advanced anyway until it reaches RePollLimit minutes before the current time. | 3600 |
| ListenUpdate | Boolean | XC Rtu will update Last Poll and Last Update field from data that is addressed to the Listen ID(s). | False |
| SaveDataItems | Boolean | XC Rtu keeps an internal real-time database of all last sensor values and statuses. These values can be spooled to disk and re-loaded on startup. This is useful when XC Rtu has display clients or HMI display data/sensor values. Values are written to RTUDATA.DAT in the XConnect directory. | False |
| LogQuality | Boolean | Log quality information based on each SSP message transmitted and received. | False |
| QualityPath | String | Directory where quality ASCII files will be stored. If using database storage, data stored in XC_RTUQC table. | [XConnect Install Dir]\Log\ |
| QualityName | String | Determines the file to append the raw NES files in to as messages are received from the mux. The lower case letters “yy”, “yyyy”, “mm”, “dd” “hh”, “mi” can be included in the name to instruct the software to create separate minute, hourly, daily, monthly, or yearly files. Optionally “#nn” can be added to specify that a new file should be created every nn minutes (when using “mi”), hours (when using “hh”), or days (all other). “#nn” can appear anywhere in the string, where nn must be two digits with leading zero (if necessary). The unit of time is determined by the smallest time specified. | RTUQLyymmdd.Log |

#### ComSetup(Port)

This function allows the user to retrieve or set Com PortSetup parameters.  This is a read-write method.  The ComSetup parameter record is defined below.  These represent the same fields as listed in the XCONNECT.INI file under  the [COMx] topic.  A port name is a required parameter for this function call.  A variant array is passed in or returned based on the ComlSetup parameter record order below.

##### COMSetup Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| Com | String | Com port name (i.e., COM1 or 600 - IP port number). | Parameter passed in |
| ComType | Enumerated | Com port type - ct_None, ct_Direct, ct_Radio, ct_Modem, ct_Virtual. | ct_None |
| Baud | Longint | Baud rate. | 9600 |
| DialRetries | Word | Number of retries when dialing fails. | 0 |
| SSPRetries | Word | Number of retries after connecting to rety SSP message. | 2 |
| NumberofRings | Word | Number of rings to send to the modem to answer incoming call. | 2 |
| MaxParsers | Word | Internal parameter - number of simultaneous SSP requests issued. | 16 |
| CarrierDelay | Longint | Time to key the radio before transmitting message (ms). | 1000 |
| NullDelay | Longint | Time to transmit Nulls before transmitting message (ms). | 0 |
| UnKeyDelay | Longint | Time to wait before unkeying radio (ms). | 500 |
| ReplyDelay | Longint | Time to wait before replying to an SSP message (ms). | 0 |
| ConnectTimeout | Longint | Time of inactivity to hold a manual connection before hanging up (ms). | 30000 |
| NoConnectTimeout | Longint | Time to wait before giving up on a connection (ms). | 60000 |
| DialTimeout | Longint | Time to wait after dialing a phone number to get a connection (ms). | 60000 |
| AutoTimeout | Longint | Time of inactivity to hold an automatic connection before hanging up (ms). | 1000 |
| AckDelay | Longint | Maximum time XC Rtu will wait for an acknowledgement (ACK) after it has sent an SSP message (ms). | 10000 |
| ResponseTimeout | Longint | Maximum XC Rtu will wait for a response from the data logger (date polls) (ms). | 30000 |
| ConnectDelay | Longint | Time to pause after a modem connection. Some modems will hang up without a delay (ms). | 0 |
| Handshake | Boolean | Use software handshaking. Used for direct connections. | True |
| HWHandshake | Boolean | Use hardware handshaking. Used for modem connections. | False |
| Touchtone | Boolean | Use touch-tone dialing. | True |
| NoWait | Boolean | Enable NoWait polling. Can be used with radio/IP polling where broadcast polling is possible. | False |
| Autobaud | Boolean | Update PC port baud rate based on telemetery device baud rate. | True |
| ControlLine | String | Sets the control line to be used in detecting connection or carrier. Valid options are: Default, None, CTS (Clear To Send), DSR (Data Send Ready), CD (Carrier Detect). If set to Default, None will be used for Direct and Virtual communication types and CD will be used for Radio and Modems. | DEFAULT |
| ModemInitString | String | Extra Hayes commands to send during modem initialization. | Blank String |
| ModemHangupString | String | Extra Hayes commands to send during modem hang up. | Blank String |
| HangupDelay | String | Time to wait before hanging up modem (ms). | 0 |
| TXPort | String | Transmit IP port number for UDP/TCP connections. | 600 |
| RXPort | String | Receive IP port number for UDP/TCP connections. | 500 |
| Protocol | String | IP protocol - UDP or TCP | UDP |
| MessageDelay | Longint | Time to delay before sending a message (ms). | 0 |

#### StationSetup(StationName)

This function allows the user to retrieve or set Station Setup parameters.  The StationSetup parameter record is defined below.  A station name is a required parameter for this function call.  A variant array is passed in or returned based on the StationSetup parameter record order below.

##### StationSetup Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| Station | String | Station name. | Parameter passed in |
| UnitID | String | RTU Unit ID. | Blank String |
| SatID | String | 8 character hexadecimal character NESDIS ID. | Blank String |
| OrbID | String | Obsolete parameter. | Blank String |
| IPAddress | String | IP address of unit. | Blank String |
| SetupFile | String | Name of setup file corresponding to RTU setup. | Blank String |
| RadioPathList | String | Comma-delimited string of primary and secondary radio paths (i.e., XPERT,RPTR_XPERT). | Blank String |
| PhoneNumberList | String | Comma-delimited string of phone numbers used in the primary and secondary communications options. | Blank String |
| UserName | String | Username used to login to the RTU. | Blank String |
| Password | String | Password used to login to the RTU. | Blank String |
| ComPortList | String | Comma-delimited string of primary and secondary communication ports (i.e., COM1,COM2 or 600,COM3). | Blank String |
| AckDelayList | String | Comma-delimited string of ACK timeouts used for the primary and secondary communications options. Normally defaults to com port settings. These station settings will override defaults (i.e., 20.30). | Blank String |
| Sensors | String | Comma-delimited string of sensor names (i.e., AT,RH,WS,WD). | Blank String |
| ModemInitString | String | Station-specific modem initialization string overrides com port setup initialization string. | Blank String |
| ModemHangupString | String | Station-specific modem initialization string overrides com port setup initialization string. | Blank String |
| SiteType | Enumerated | RTU site type - st_None, st_8200, st_8210, st_XLite, st_XPert, st_Satlink, st_9000, st_8004, st_PC, st_OTHER, st_ALERT. | st_XLite |
| PollType | Enumerated | Station specific poll type that is used when a poll group is defined and poll group poll type is set to None. pt_None, pt_Date, pt_Newest, pt_Current, pt_NoWait | pt_None |
| Enable | Boolean | Enable indicates active station that will be polled. | False |
| LoginReqd | Boolean | Login required to RTU before data request (normally used with modems). | False |
| QuickProto | Boolean | Quick protocol used. All current loggers use auto-protocol. Used for SSP communications. | True |
| AutoProto | Boolean | RTU com port is configured in SSP mode. | True |
| LastPoll | Longint | Seconds since 12/31/1984 representing the last date/time of data for a Date Poll. | Defaults to current date/time. |
| Concentrate | Boolean | RTU is used as a concentrator of data for other RTUs. | False |
| HoursAhead | Single | Hours to add to the received date/time of data received an SSP messages. Used by XC Rtu only. | 0 |
| RDIBaseTime | Longint | Seconds since 12/31/1984 representing the base time for the RDI-enabled RTUs to turn power to the RDI radio. Used for EDACS systems only. | 0 = 00:00:00 |
| RDIInterval | Longint | Seconds since 12/31/1984 representing the interval for the RDI-enabled RTUs to turn power to the RDI radio. Used for EDACS systems only. | 0 = 00:00:00 |
| RDIDwell | Longint | Seconds since 12/31/1984 representing the stay on/power on time for the RDI-enabled RTUs to turn power to the RDI radio. Used for EDACS systems only. | 0 = 00:00:00 |
| TemplateName | String | Name of station template. | Blank String |
| Status | Enumerated | Status of station - ss_None, ss_Normal, ss_Offline, ss_Maintenance, ss_Discontinued, ss_Future. | ss_None |
| SiteLongName | String | Optional longer name for station. | Blank String |
| Site Code | String | Code for station. | Blank String |
| USGSID | String | USGS ID for station. | Blank String |
| Reservoir | String | Reservoir station is located. | Blank String |
| RiverMile | Single | River mile location of station. | 0 |
| SiteComment | String | User comment for station. | Blank String |
| InstallDate | Longint | Installation date for sensor. | 0 - 00/00/0000 00:00:00 |
| Agency | String | Agency that owns station. | Blank String |
| Country | String | Country where station is located. | Blank String |
| District | String | District where station is located. | Blank String |
| City | String | City where station is located. | Blank String |
| State | String | State where station is located. | Blank String |
| County | String | County where station is located. | Blank String |
| Basin | String | Basin where station is located. | Blank String |
| Latitude | Single | Latitude of station location. | Blank String |
| Longitude | Single | Longitude of station location. | Blank String |
| Elevation | Single | Elevation of station. | Blank String |
| Coefficient1 | Single | Coefficient 1, used in optional SNS_COEFF equation function. | 0 |
| Coefficient2 | Single | Coefficient 2, used in optional SNS_COEFF equation function. | 0 |
| Coefficient3 | Single | Coefficient 3, used in optional SNS_COEFF equation function. | 0 |
| PrimaryChannel | Integer | Channel for self-timed satellite transmissions. | 0 |
| RandomChannel | Integer | Channel for random satellite transmissions. | 0 |
| Reporting Time | Longint | Seconds since 12/31/1984 representing the time of satellite data transmission. | 0 = 00:00:00 |
| ReportingInterval | Longint | Seconds since 12/31/1984 representing the interval of satellite transmissions. | 0 = 00:00:00 |
| LastUpdate | Longint | Seconds since 12/31/1984 representing the date and time of last data received from station. | 0 = 00/00/0000 00:00:00 |
| TZoneCode | String | Time zone code. | Blank String |
| GMTOffset | Integer | GMT offset of station location. | 0 |
| SHEFID | String | SHEF ID of station. | Blank String |
| AltCharID1 | String | Alternate ID 1. | Blank String |
| AltCharID2 | String | Alternate ID 2. | Blank String |
| GPString1 | String | General purpose string 1. | Blank String |
| GPString2 | String | General purpose string 2. | Blank String |
| GPNumber1 | Single | General purpose number 1 used in STN_GPN equation function. | 0 |
| GPNumber2 | Single | General purpose number 2 used in STN_GPN equation function. | 0 |
| PicFileName | String | Name of picture file corresponding to station setup/location. | Blank String |
| WeightFactor | Single | Weighting factor applied for station. | 0 |

#### SensorSetup(StationName, SensorName)

This function allows the user to retrieve or set Sensor Setup parameters.  The SensorSetup parameter record is defined below.  Two parameters are required for the function call: station name and sensor name.  A variant array is passed in or returned based on the SensorSetup parameter record order below.

##### SensorSetup Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| Station | String | Station name. | Parameter passed in |
| Sensor | String | Sensor name. | Parameter passed in |
| Equation | String | Equation applied to sensor value. | Blank String |
| TagName | String | Name of sensor as defined in the RTU. | Blank String |
| RecordID | String | Comma-delimited string of sensor names representing the order of numbers returned from a Date/Time tag file from an 8200/8210 or 9210 using EZSetup. | Blank String |
| Raw | String | Comma-delimited list of sensor to forward the value of the sensor. | Blank String |
| Enable | Boolean | Enable indicates active sensor that will be parsed and stored. | False |
| RightDigits | Word | Right digits scaling for sensor; should match right digits setting in RTU. | 2 |
| IsText | Boolean | Flag indicating data from 9210/XPert is text and not numerical. | False |
| RoundOffTime | Longint | Seconds since 12/31/1984 representing the time to round off of the acutual sensor time (i.e. 00:15:00 - all times will be rounded to a 15 minute interval. | 0 = 00:00:00 |
| DataStoreInterval | Longint | Seconds since 12/31/1984 representing the time of data to be stored (i.e., 00:15:00 - only data collected at 15 minute intervals will be stored). | 0 = 00:00:00 |
| AlertID | Integer | Numerical ALERT ID for sensor. | 0 |
| AlertType | Enumerated | ALERT data type - alt_None, alt_Normal, alt_WindRun, alt_WindDir. | alt_None |
| TemplateName | String | Name of sensors template. | String |
| LeftDigits | Word | Left digits for sensor; used for formatting in XC Export. | 1 |
| Units | String | Units associated with sensor (i.e., mbar, deg). | Blank String |
| Status | Enumerated | Status of sensor - ss_None, ss_Normal, ss_Offline, ss_Maintenance, ss_Discontinued, ss_Future. | ss_None |
| Reference1 | Single | Reference value for sensor (i.e., lower bound value). | 0.0 |
| Reference2 | Single | Reference value for sensor (i.e., upper bound value). | 0.0 |
| DataTable | String | Name of data table where sensor data is stored. | XC_DATA1 |
| Description | String | Description/comment of sensor. | Blank String |
| Manufacturer | String | Manufacturer of sensor. | Blank String |
| ModelNumber | String | Model number of sensor. | Blank String |
| SHEFPECode | String | SHEF code for sensor. | Blank String |
| ParamCode | String | Parameter code for sensor. | Blank String |
| AltParamCode1 | String | Alternate parameter 1 code for sensor. | Blank String |
| AltParamCode2 | String | Alternate parameter 2 code for sensor. | Blank String |
| RatingTable | String | Rating table file name when using DBTABLE equation function. | 0 |
| Offset | Single | Value used when using OFFSET equation function. | 0 |
| Shift | Single | Value used when using SHIFT equation function. | 0 |
| HighHighLimit | Single | High high threshold value. | 0 |
| HighHighAlarm | Enumerated | Alarm evaluation type = al_None, al_Above, al_Below. | al_None |
| HighLimit | Single | High threshold value. | 0 |
| HighAlarm | Enumerated | Alarm evaluation type = al_None, al_Above, al_Below. | al_None |
| LowLimit | Single | Low threshold value. | 0 |
| LowAlarm | Enumerated | Alarm evaluation type = al_None, al_Above, al_Below. | al_None |
| LowLowLimit | Single | Low low threshold value. | 0 |
| LowLowAlarm | Enumerated | Alarm evaluation type = al_None, al_Above, al_Below. | al_None |
| ROCLimit | Single | Rate-of-Change threshold value. | 0 |
| ROCAlarm | Enumerated | Alarm evaluation type = al_None, al_Above, al_Below. | al_None |
| ROCInterval | Longint | Time duration to evaluate rate-of-change (seconds). | 0 = 00:00:00 |
| Deadband | Single | Deadband for ROC alarm. | 0.0 |
| NoChangeInterval | Longint | Time duration to evaluate No change condition. | 0 = 00:00:00 |
| NoChangeAlarm | Enumerated | Alarm evaluation type = al_None, al_Above, al_Below. | al_None |
| TextLimit | String | Text alarm. | Blank String |
| TextAlarm | Enumerated | Alarm evaluation type = al_None, al_Above, al_Below. | al_None |
| StdDevLimit | Single | Standard deviation threshold value. | 0.0 |
| StdDevAlarm | Enumerated | Alarm evaluation type = al_None, al_Above, al_Below. | al_None |
| StdDevInterval | Longint | Time duration to evaluate standard deviation. | 0 = 00:00:00 |
| AvgLimit | Single | Average threshold value. | 0.0 |
| AvgAlarm | Enumerated | Alarm evaluation type = al_None, al_Above, al_Below. | al_None |
| AvgInterval | Longint | Time duration to evaluate average. | 0 = 00:00:00 |
| Coefficient1 | Single | Coefficient 1 value used in SNS_COEFF equation function. | 0.0 |
| Coefficient2 | Single | Coefficient 2 value used in SNS_COEFF equation function. | 0.0 |
| Coefficient3 | Single | Coefficient 3 value used in SNS_COEFF equation function. | 0.0 |
| Coefficient4 | Single | Coefficient 4 value used in SNS_COEFF equation function. | 0.0 |
| Coefficient5 | Single | Coefficient 5 value used in SNS_COEFF equation function. | 0.0 |
| GPNumber1 | Single | General purpose number 1 used in SNS_GPN equation function. | 0.0 |
| GPNumber2 | Single | General purpose number 2 used in SNS_GPN equation function. | 0.0 |
| GPString1 | String | General purpose string 1. | Blank String |
| GPString2 | String | General purpose string 2. | Blank String |
| UniqueKey | String | Custom sensor key used with custom databases to identify the station and sensor. | Blank String |

#### SensorDecodeSetup(Topic, StationName, SensorName)

This function allows the user to retrieve or set Sensor Decode Setup parameters.  The SensorDecodeSetup parameter record is defined below.  Three parameters are required for the function call: topic, station name and sensor name.  A variant array is passed in or returned based on the SensorDecodeSetup parameter record order below.

##### SensorSetup Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| Topic | String | Decoding topic (i.e., GOESTIMED, GOESRANDOM) | Parameter passed in |
| Station | String | Station name. | Parameter passed in |
| Sensor | String | Sensor name. | Parameter passed in |
| Enable | Boolean | Enable indicates active sensor that will be decoded and stored. | False |
| Format | String | Transmission format (i.e., 6BIT, 6UBIT, ASCII) | 6BIT |
| SeqNum | Integer | Used if more that one decoding profile for same sensor in message. | 0 |
| RelativeTime | Boolean | Times are relative to where the previous decode left off. | False |
| RelativePos | Boolean | Positions are relative to where the previous decode left off. | False |
| AddBase | Boolean | Data values are relative to where offsets from the last the previous decode left off. | False |
| ByteSize | Word | Size in bytes of the encoded sensor value. | 3 |
| BitSize | Word | Size in bits of encode sensor value. | 0 |
| Bound1 | Integer | Start position in the message after the NESDIS header to look for the optional Block ID. | 0 |
| Bound2 | Integer | End position in the message after the NESDIS header to look for the Block ID. | 0 |
| BlockID | String | There may be a label or unique set of characters that precede the location where the sensor values. | Blank String |
| BytePos | Integer | Byte position in message to start reading characters for sensor value. | 1 |
| BitPos | Integer | Bit position in message to start reading characters for sensor value. | 1 |
| Next | Integer | Distance in bytes from the BytePos position to the next occurrence of the sensor value. | 3 |
| BitNext | Integer | Distances in bits from the BitPos position to the next occurrence of the sensor value. | 1 |
| Amount | Word | Number of values encoded for the sensor. | 0 |
| DeltaTime | Longint | Seconds since 12/31/1984 representing the time difference in measurement times between the multiple sensor values (seconds). | 900 = -00:15:00 |
| OffsetTime | Longint | Seconds since 12/31/1984 representing the time difference between the measurement time of first decoded sensor value and time of transmission as indicated in the NESDIS header. | 0 = 00:00:00 |
| RoundOffTime | Longint | Round off the time stamp calculated by the Interval and Offset to a desired minute or hour. | 0 = 00:15:00 |
| SkipBlank | Word | Number of blanks to skip to look for sensor position. | 0 |
| SkipComma | Word | Number of commas to skip to look for sensor position. | 0 |
| SkipLF | Word | Number of LF to skip to look for sensor position. | 0 |
| IgnoreCase | Boolean | Ignore case-sensitivity when look for Block ID. | False |
| More | String | Indicates if another decoding profile exists for sensor | Blank String |
| TemplateName | String | Name of sensor decoding template. | Blank String |

#### DecoderSetup

This function allows the user to retrieve or set XC Decode Setup parameters.  The DecoderSetup parameter record is defined below.  These represent the same fields as listed in the XCONNECT.INI file under the [Decoder] topic.

##### DecoderSetup Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| Run | String | Comma-delimited list of applications that will be started when XC Decode starts (i.e., XCAlarm.exe,Excel.exe,Display.exe). | Blank String |
| Clients | String | Comma-delimited list of applications that must be running before XC Decode starts to deocde raw messages. Must use the Windows application title, not the executable name. | Blank String |
| AutoPath | String | Directory path where raw files are stored. | Blank String |
| AutoBaseTime | Longint | Seconds since 12/31/1984 representing the base time to check for new satellite messages in raw files. | 0 = 00:00:00 |
| AutoInterval | Longint | Seconds since 12/31/1984 representing the interval to check for new satellite messages in raw files. | 3600 = 01:00:00 |
| LogQuality | Boolean | Log satellite quality information from each message. | False |
| QualityPath | String | If NOT using database data storage, directory location to store ASCII log files. | [XConnect Install Dir]\Log\ |
| QualityName | String | Naming pattern for quality files. | QLyymmdd.Log |
| SaveDataItems | Boolean | XC Decode keeps an internal real-time database of all last sensor values and statuses. These values can be spooled to disk and re-loaded on startup. This is useful when XC Decode has display clients or HMI display data/sensor values. Values are written to DECODEDATA.DAT in the XConnect directory. | False |

#### MuxSetup

This function allows the user to retrieve or set XC Mux Setup parameters.  The MuxSetup parameter record is defined below.  These represent the same fields as listed in the XCONNECT.INI file under the [Mux] topic.

##### MuxSetup Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| Topic | String | Use if more than one XC Mux is running. Currently unused. | Blank String |
| Run | String | Comma-delimited list of applications that will be started when XC Mux starts (i.e., XCAlarm.exe,Excel.exe,Display.exe). | Blank String |
| ComPort | String | Com port for DDRGS connection. Not used with current Sutron DRGS model (DSR DRGS). | Blank String |
| DDRGSType | Enumerated | DDRGS model type - dgt_None, dgt_100Baud, dgt_HDRDDRGS, dgt_DSRDRGS. | dgt_None |
| Baud | Word | Baud rate for serial port-DDRGS connection. | 9600 |
| RawPath | String | Directory path where raw files will be stored. | [XConnect Install Dir]\RAw\ |
| RawFile | String | Naming pattern for raw files. | yyymmdd.Raw |
| Reset | String | Commands sent to DDRGS when issuing a Reset command. Used only for serial port DDRGS models. | Blank String |
| Bound1 | Integer | Obsolete parameter. | 0 |
| Bound2 | Integer | Obsolete parameter. | 60 |
| Listen | Boolean | If two PCs are connected to the same serial port DDRGS , then only one can be considered the primary controlling PC. The other PC, secondary, will have XC Mux configured to Listen mode, receiving and storing messages, but not controlling the DDRGS. | False |
| TimeOffset | Single | Offset to add to time command. Used on with serial port DDRGS models. | 0 |
| NumRawCopies | Integer | Number of copies of raw files. | 0 |
| SaveDataItems |  | XC Mux keeps an internal real-time database of all last sensor values and statuses. These values can be spooled to disk and re-loaded on startup. This is useful when XC Mux has display clients or HMI display data/sensor values. Values are written to MUXDATA.DAT in the XConnect directory. | False |
| IPAddress | String | IP Address of DSR DRGS model. | Blank String |
| IPPort | String | IP Port of DSR DRGS model. | 5010 |
| AddETX | Boolean | Add three ETX characters to each raw messages. Used for other decoding programs. | False |
| Add4Bytes | Boolean | Increase Length value in NESDIS header by 4. | False |
| AddEOT | Boolean | Add space representing EOT byte. | False |
| ForceTimeSync | Boolean | Always perform a time sync to DDRGS every hour. Only used for serial port DDRGS models. | False |
| NoNESDISHdr | Boolean | Write raw messages without the NESDIS header. | False |

#### DAPSSetup

This function allows the user to retrieve or set XC Daps Setup parameters.  The DAPSSetup parameter record is defined below.  These represent the same fields as listed in the XCONNECT.INI file under the [DAPSDialer] topic.

##### DAPSSetup Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| Run | String | Comma-delimited list of applications that will be started when XC DAPS starts (i.e., XCAlarm.exe,Excel.exe,Display.exe). | Blank String |
| IPAddress | String | DAPS telnet address. | 205.156.2.173 |
| RawPath | String | Directory path where raw files will be stored. | [XConnect Install Dir]\Raw\ |
| RawFile | String | Naming pattern for raw files. | yyymmdd.Raw |
| User | String | Username for login to DAPS site. | Blank String |
| Password | String | Password for login to DAPS site. | Blanks String |
| FirewallPass | String | Firewall password. | 60 |
| BaseTime | Longint | Seconds since 12/31/1984 representing the base time to log into DAPS and download new satellite messages. | 0 = 00:00:00 |
| Interval | Longint | Seconds since 12/31/1984 representing the interval to log into DAPS and download new satellite messages. | 0 = 00:00:00 |
| Retries | Word | Number of time to retry if DAPS login fails. | 2 |
| RetryWait | Longint | Seconds to wait in between retries. | 5 |
| MsgTypes | String | Optional DAPS flags (i.e., G, ?, W, A, D, T, U, M, N) | Blank String |
| SaveDataItems | Boolean | XC DAPS keeps an internal real-time database of all last sensor values and statuses. These values can be spooled to disk and re-loaded on startup. This is useful when XC DAPS has display clients or HMI display data/sensor values. Values are written to DAPSDATA.DAT in the XConnect directory. | False |
| NoNESDISHdr | Boolean | Write raw messages without the NESDIS header. | False |
| LastDownload | Longint | Seconds since 12/31/1984 representing the last date/time of satellite message downloaded | Defaults to current date/time. |
| Stations | String | Comma-delimited string of stations to download, if all stations were not selected | Blank String |

#### PollGroupSetup(PollNumStr)

This function allows the user to retrieve or set Poll Group Setup parameters used by XC Rtu.  The PollGroupSetup parameter record is defined below.  These represent the same fields as listed in the XCONNECT.INI file under the [PollGroup:x] topic.  One parameter are required for the function call: pollgroup number.

##### PollGroupSetup Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| PollGroup | Word | Poll group number (i.e., 1,2,3...) | Derived from parameter passed in. |
| PollGroupStr | String | Poll group number as a string. | Parameter passed in |
| Stations | String | Comma-delimited string of stations to poll. | Blank String |
| Clients | String | Comma-delimited list of applications that must be running before XC Rtu triggers the poll group. Must use the Windows application title, not the executable name. | Blank String |
| Control | String | Additional control command to be sent to RTU | Blank String |
| ControlOnly | Boolean | Only issue the control command and not the poll for the poll group. | False |
| Enable | Boolean | Enable indicates active poll group that will be triggered. | False |
| BaseTime | Longint | Seconds since 12/31/1984 representing the base time for XC Rtu to trigger poll group. | 0 = 00:00:00 |
| Interval | Longint | Seconds since 12/31/1984 representing the interval for XC Rtu to trigger poll group. | 900 = 00:15:00 |
| TimeSync | Enumerated | Time sync options - ts_None, ts_No, ts_Yes, ts_Only, ts_All. | ts_None |
| TimeLimit | Longint | Maximum amount of data XC Rtu will download from an RTU. | 0 = 00:00:00 |
| PollType | Enumerated | Poll type options - pt_None, pt_Date, pt_Newest, pt_Current, pt_NoWait. | pt_None |
| OverrideOrder | Boolean | Override auto-ordering used by XC Rtu. | False |
| Description | String | Description of poll group. | Blank String |
| TimeSyncPort | String | Com port to broadcast SSP time sync command (i.e., COM1). | None |
| LogFile | String | Log file in RTU to retrieve data. | SSP.LOG |
| LogFilePath | Longint | Location of file in RTU (i.e., \FLASH DISK\ or \STORAGE DISK\). | \FLASH DISK\ |
| Owner | String | Owner of poll group. Typically PC name, not Master ID. | Blank |

#### DataServerSetup

This function allows the user to retrieve or set general Data Storage Setup parameters.  This is a read-write method.  The general Data Storage Setup parameter record is defined below.  These represent the same fields as listed in the XCONNECT.INI file under  the [DataServer] topic.

##### DataServerSetup Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| StoreGroups | String | Comma-delimited list of data store groups created (i.e., 1,2,3). | Blank string |
| DefaultDB | String | Name of default ODBC data source | Blank string |
| DefaultUser | String | User name for login to DefaultDB | Blank string |
| DefaultPass | String | Password for login to DefaultDB | Blank string |
| DefaultDataSource | Enumerated | Data storage type - dt_None, dt_PcBase, dt_Oracle, dt_Logger, dt_Access, dt_EXCEL, dt_XML, dt_SQLSrvr, dt_HDB, dt_InSQL. | dt_NOne |

#### DataStoreGroupSetup(StoreStr)

This function allows the user to retrieve or set Data Store Group Setup parameters used by XC DataStore.  The DataStoreGroupSetup parameter record is defined below.  These represent the same fields as listed in the XCONNECT.INI file under the [DataStoreGroup:x] topic.  One parameter are required for the function call: storegroup number.

##### DataStoreGroupSetup Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| StoreGroup | Word | Store group number (i.e., 1,2,3...) | Derived from parameter passed in. |
| StoreGroupStr | String | Poll group number as a string. | Parameter passed in |
| Enable | Boolean | Enable indicates active data store group. | False |
| DataPath | String | Directory location of PcBase2 data files when PcBase2 data storage type selected. | Blank String |
| DataFiles | String | Comma-delimited string of PcBase2 data file names. | Blank String |
| DataSource | Enumerated | Data storage type - dt_None, dt_PcBase, dt_Oracle, dt_Logger, dt_Access, dt_EXCEL, dt_XML, dt_SQLSrvr, dt_HDB, dt_InSQL. | dt_None |
| ODBCSource | String | ODBC Data Source name when database type storage selected. | Blank String |
| ODBCUser | String | Login user name for database. | Blank String |
| ODBCPass | String | Login password for database. | Blank String |
| DataLimit | Longint | Number of days of data to maintain. Other data will be deleted. Not valid for SQL Server, Oracle, HDB or INSQL options. | 0 |
| Timetag | String | All data from Date and Manual polls will be stored, or only from a comma-delimited list of specific stations. | * |
| RealTime | String | All data from Current polls will be stored, or only from a comma-delimited list of specific stations. | * |
| LogPath | String | Directory location of ASCII files when Logger, Excel, XML storage type is selected. | [XConnect Install Dir]\Log\ |
| LogOutputFile | String | Naming pattern for ASCII files | yyyymmdd.Log |
| AppendName | Enumerated | Currently unused - ant_None, ant_Prefix, ant_Suffix, ant_Both. | ant_None |
| Description | String | Description of data store group | Blank String |
| StoreAll | Boolean | Currently unused. Future parameter. | False |
| DoSystemLog | Boolean | Currently unused. Future parameter. | False |

#### DeleteTopicItem(Topic, Item)

This procedure allows the user to delete a key in the XConnect.INI file  Two parameters are required for the function call: Topic and Item.  The Topic is the parameter in the INI file surrounded by brackets such as [General].  The Item is the parameter entry in the topic, such as Use24Hour.  As a note, station, sensor and poll group information should not be deleted.  This information is stored in the database and will overwrite and user changes.

#### AddTopicItem(Topic, Item, Data)

This procedure allows the user to add a key in the XConnect.INI file  Three parameters are required for the function call: Topic, Item and Data.  The Topic is the parameter in the INI file surrounded by brackets such as [General].  The Item is the parameter entry in the topic, such as ImagePath.  The Data is the value, such as C:\Windows.  As a note, station, sensor and poll group information should not be added.  This information is stored in the database and will overwrite and user changes.

#### SingleItem(Item, Data)

This function allows the user to retrieve or set a single value from the internal real-time database.  Two parameters are required for the function call: Topic and Item.  The Topic is the parameter in the INI file surrounded by brackets such as [General].  The Item is the parameter entry in the topic, such as Use24Hour.  As a note, station, sensor and poll group information should not be deleted.  This information is stored in the database and will overwrite and user changes.

#### ResetAllParams(AppGUIDStr)

This procedure allows the user to issues a software reset to all XConnect applications that retrieve information from XC Setup.  One parameter is required for the procedure call:  Application GUID string.  For the XConnect applications, when a ResetAllParams command is received, they will stop all processing, close any com ports and the re-request the latest setup information from XC Setup.

#### AppShutdown(AppGUIDStr)

This procedure will broadcast an application shutdown command to XConnect applications.  One parameter is required for the procedure call:  Application GUID string.  This method All XConnect servers and processing applications will respond to this command.  Applications that strictly connect to the database such as XC DataView, XC Reports or XC Export will not.

#### SetItemLink(AppGUIDStr, Item)

This procedure allows a client application to set a link to a specific data item.  XC Setup will send out an event to all clients via their GUID string with the new value.  Two parameters are required for this procedure call:  Application GUID string and item.  An item could be a specific sensor value such as XPERT.BATTERY.

#### StationListDetails(StationName)

This function returns a variant record of station details.  One parameter is required for the function call:  StationaName.  XC Desktop uses this function call to when displaying the Station List view.

##### StationDetails Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| Name | String | Station Name | Parameter passed in. |
| SiteType | String | None, 8200, 8210, 9210, XPert, SATLINK, 9000, 8004, PC, OTHER, ALERT. | NONE |
| Connection | String | Connection port (COM1 or DDRGS). | Current date and time |
| DateModified | String | Last time parameters were modified (mm/dd/yyyy hh:mm:ss). | Blanks string |
| Enable | Boolean | Station enabled. | False |

#### RtuPortListDetails(Comport)

This function returns a variant record of com port details for XC Rtu.  One parameter is required for the function call:  com port (i.e., COM1).  XC Desktop uses this function call to when displaying the Communication Port List view.

##### RtuPortDetails Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| Name | String | Communication port | Parameter passed in. |
| Type | String | NONE, DIRECT, RADIO, MODEM, VIRTUAL. | NONE |
| DateModified | String | Last time parameters were modified (mm/dd/yyyy hh:mm:ss). | Current date and time |
| Baud | String | Connection baud rate. | 0 |

#### MuxPortListDetails(Comport)

This function returns a variant record of com port details for XC Mux.  One parameter is required for the function call:  com port (i.e., COM1).  XC Desktop uses this function call to when displaying the Communication List view.

##### MuxPortDetails Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| Name | String | Communication port | Parameter passed in. |
| Type | String | NONE, 100-BPS DDRGS, HDR DDRGS, DSR DRGS. | NONE |
| DateModified | String | Last time parameters were modified (mm/dd/yyyy hh:mm:ss). | Current date and time |
| Baud | String | Connection baud rate. | 0 |

#### PollGroupListDetails

This function returns a variant record of poll group details for XC Rtu.  One parameter is required for the function call:  Poll group number (i.e., PollGroup:1).  XC Desktop uses this function call to when displaying the Poll Group List view.

##### PollGroupDetails Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| Name | String | Poll group name. | Parameter passed in. |
| Poll Type | String | NONE, DATE, NEWEST, CURRENT, NOWAIT. | NONE |
| BaseTime | String | Base time of poll group (hh:mm:ss). | 00:00:00 |
| Interval | String | Interval of poll group (hh:mm:ss). | 00:00:00 |
| Description | String | Poll group description. | Blank string |
| DateModified | String | Last time parameters were modified (mm/dd/yyyy hh:mm:ss). | Current date and time |
| Enable | Boolean | Poll group enabled. | False |

#### DataStoreGroupListDetails

This function returns a variant record of data store group details for XC DataStore.  One parameter is required for the function call:  Poll group number (i.e., DataStoreGroup:1).  XC Desktop uses this function call to when displaying the Data Store Group List view.

##### DataStoreGroupDetails Record

| Parameter Name | Type | Description | Default |
| --- | --- | --- | --- |
| Name | String | Data storel group name. | Parameter passed in. |
| Poll Type | String | NONE, PCBASE, ORACLE, LOGGER, ACCESS, EXCEL, XML, SQLSERVER, HDB, INSQL. | NONE |
| Path | String | Path of data storage. For database storage, use ODBC name, for ASCII file storage, directory path. | 00:00:00 |
| DateModified | String | Last time parameters were modified (mm/dd/yyyy hh:mm:ss). | Current date and time |
| Enable | Boolean | Data store group enabled. | False |

#### DeleteStation(StationName)

This procedure allows the user to delete a station from the default database and XC Setup internal real-time database.  One parameter is required for the procedure call: StationName.  If more than on data storage option is used, this procedure call will only delete from the default/primary database.

#### DeleteSensor(StationName, SensorName)

This procedure allows the user to delete a sensor from the default database and XC Setup internal real-time database.  Two parameters are required for the procedure call: Station name and sensor name.  If more than on data storage option is used, this procedure call will only delete from the default/primary database.

#### DeleteComPort(Comport)

This procedure allows the user to delete a com port from the INI file and XC Setup internal real-time database.  One parameter is required for the procedure call: port name/number (i.e., COM1, 600).

#### DeletePollGroup(PollgroupStr)

This procedure allows the user to delete a poll group from the default database and XC Setup internal real-time database.  One parameter is required for the procedure call: poll group string.  If more than on data storage option is used, this procedure call will only delete from the default/primary database.

#### DeleteDataStoreGroup(DataGroupStr)

This procedure allows the user to delete a data store group from the INI file and XC Setup internal real-time database.  One parameter is required for the procedure call: data store group string.

#### AddStation(StationName)

This procedure allows the user to add a station from the default database and XC Setup internal real-time database.  One parameter is required for the procedure call: StationName.  If more than on data storage option is used, this procedure call will only delete from the default/primary database.  This call creates a station with the default parameters.  A second call to update the station setup (StationSetup) should be made with user-selected parameters.

#### AddSensor(StationName, SensorName)

This procedure allows the user to add a sensor from the default database and XC Setup internal real-time database.  Two parameters are required for the procedure call: Station name and sensor name.  If more than on data storage option is used, this procedure call will only delete from the default/primary database.  This call creates a sensor with the default parameters.  A second call to update the sensor setup (SensorSetup) should be made with user-selected parameters.

#### AddComPort(Port)

This procedure allows the user to add a com port from the INI file and XC Setup internal real-time database.  One parameter is required for the procedure call: port name/number.  This call creates a com port with the default parameters.  A second call to update the com port setup (ComSetup) should be made with user-selected parameters.

#### AddPollGroup(PollgroupStr)

This procedure allows the user to add a poll group from the default database and XC Setup internal real-time database.  One parameter is required for the procedure call: poll group string.  If more than on data storage option is used, this procedure call will only add to the default/primary database.  This call creates a poll group with the default parameters.  A second call to update the poll group setup (PollGroupSetup) should be made with user-selected parameters.

#### AddDataStoreGroup(DataGroupStr)

This procedure allows the user to add a data store group from the INI file and XC Setup internal real-time database.  One parameter is required for the procedure call: data store group string. This call creates a data store group with the default parameters.  A second call to update the data store group setup (DataStoreGroupSetup) should be made with user-selected parameters.

#### ResetMuxParms(AppGUIDStr)

This procedure allows the user to issues a reset to all XConnect applications that use the XC Mux setup parameters.  One parameter is required for the procedure call: application GUID string of application originating procedure call.  Currently, XC Mux will call this procedure when the user has changed the XC Mux setup parameters from XC Mux and not from XC Desktop.  This helps all applications in sync when parameters can be modified from multiple applications.

#### ResetDecoderParms(AppGUIDStr)

This procedure allows the user to issues a reset to all XConnect applications that use the XC Decode setup parameters.  One parameter is required for the procedure call: application GUID string of application originating procedure call.  Currently, XC Decode will call this procedure when the user has changed the XC Decode setup parameters from XC Decode and not from XC Desktop.  This helps all applications in sync when parameters can be modified from multiple applications.

#### ResetRtuServerParms(AppGUIDStr)

This procedure allows the user to issues a reset to all XConnect applications that use the XC Rtu setup parameters.  One parameter is required for the procedure call: application GUID string of application originating procedure call.  Currently, XC Rtu will call this procedure when the user has changed the XC Rtu setup parameters from XC Rtu and not from XC Desktop.  This helps all applications in sync when parameters can be modified from multiple applications.

#### ResetSensorDecodeParms(AppGUIDStr)

This procedure allows the user to issues a reset to all XConnect applications that use the sensor decode setup parameters.  One parameter is required for the procedure call: Application GUID string of application originating procedure call.  Currently, XC Decode will call this procedure when the user has changed the sensor decode setup parameters from XC Decode Interactive Decoding window and not from XC Desktop.  This helps all applications in sync when parameters can be modified from multiple applications.

#### ResetDAPsParms(AppGUIDStr)

This procedure allows the user to issues a reset to all XConnect applications that use the XC Daps setup parameters.  One parameter is required for the procedure call: Application GUID string of application originating procedure call.  Currently, XC Daps will call this procedure when the user has changed the XC Daps setup parameters from XC Daps and not from XC Desktop.  This helps all applications in sync when parameters can be modified from multiple applications.

#### LoadSetup(FilenName, NewFile)

This procedure allows the user to load a different/new INI file.  This is the same as selecting File|Open from the XC Setup main window.  Two parameters are required for the procedure call: Filename and boolean flag to indicated whether this is a new INI file.

#### AssignTemplate

This procedure is currently unused.

#### UnAssignTemplate

This procedure is currently unused.

#### FromTemplate

This procedure is currently unused.

#### RenameComPort(OldPort, NewPort)

This procedure allows the user to rename an existing com port to another.  Two parameters are required for the procedure call:  Old port and new port.

#### SaveINIFile

This procedure allows the user to load a save the XC Setup internal settings to the INI file.  This is the same as selecting File|Save from the XC Setup main window.  By default, the settings are saved to disk every 10 minutes.

#### SetItemChange(AppGUIDStr, Item, Data)

This procedure allows an application to broadcast an message for a data item value changed.  This provides data to any client applications with hot-links to data items.  Three parameters are required for the procedure call:  Application GUID string, Item, Data.

#### UserLoginChange(AppGUIDStr, User, Access)

This procedure allows the user to instruct XC Setup to broadcast an change in the current user logged into XConnect.  Users are optional.  Each XConnect application has a Login option.  This is NOT the database login.  Three parameters are required for the procedure call:  Application GUID string (application originating procedure call), User name and User Access string.

#### IsAlive(TimeString)

This procedure broadcasts a call with the current date time string (yyyy/mm/dd hh:mm:ss) to indicate the application is functioning properly and not frozen.

#### CurrentUser(User, Access)

This procedure allows a client application to request the current user logged in to the XConnect applications

#### ResetDBParms(AppGUIDStr)

This procedure allows the user to instruct XC Setup to re-read the station, sensor and poll group information in the database.  Next it will trigger a ResetAllParams call for all XConnect application refresh its setup information.  One parameter is required for the procedure call: Application GUID string of application originating procedure call.

#### ResetDBPollGroups(AppGUIDStr)

This procedure allows the user to instruct XC Setup to re-read the poll group information in the database.  Next it will trigger a ResetAllParams call for all XConnect application refresh its setup information.  One parameter is required for the procedure call: Application GUID string of application originating procedure call.

### Events

#### OnResetParams(AppGUIDStr)

This event notifies XConnect client applications when setup information has been changed.  Applications such as XC Rtu, XC Decode, XC Alarm or XC Calc will stop the current processing, close ports, request setup information from XC Setup and lastly, re-start processing.

#### OnResetAllParams(AppGUIDStr)

This event notifies XConnect client applications when setup information has been changed.  Applications such as XC Rtu, XC Decode, XC Alarm or XC Calc will stop the current processing, close ports, request setup information from XC Setup and lastly, re-start processing.

There is no longer a difference between OnResetParams and OnResetAllParams events.

#### OnAppShutdown(AppGUIDStr)

This event notifies XConnect client applications when XC Setup will shutdown.  Many applications such as XC Rtu, XC Decode, XC Alarm or XC Calc require XC Setup, thus when XC Setup shuts down, they will also shutdown.

#### OnResetCOMParams(AppGUIDStr)

This event is obsolete.

#### OnItemChange(AppGUIDStr, Item, Data)

This event notifies XConnect client applications when a data item has changed.  This provides data to any client applications with hot-links to data items.  Three parameters are passed with the event:  Application GUID string, Item, Data.

#### OnMuxParamsChange(AppGUIDStr)

This event notifies XConnect applications when XC Mux setup parameters have changed.  Thus, client applications can request updated XC Mux setup parameters.

#### OnDecoderParamsChange(AppGUIDStr)

This event notifies XConnect applications when XC Decod setup parameters have changed.  Thus, client applications can request updated XC Decode setup parameters.

#### OnRtuServerParamsChange(AppGUIDStr)

This event notifies XConnect applications when XC Rtu setup parameters have changed.  Thus, client applications can request updated XC Rtu setup parameters.

#### OnSensorDecodeParams(AppGUIDStr)

This event notifies XConnect applications when sensor decoding setup parameters have changed.  Thus, client applications can request updated decoding setup parameters.

#### OnDAPSParamsChange(AppGUIDStr)

This event notifies XConnect applications when XC Daps setup parameters have changed.  Thus, client applications can request updated XC Daps setup parameters.

#### OnUserLoginChange(AppGUIDStr)

This event notifies XConnect applications when the XConnect user has changed.  Users are optional.  Thus, client applications can adjust privileges to setup windows when users change.

#### OnHeartBeat(AppGUIDStr, TimeString)

This event notifies XConnect applications of the application heartbeat.  Each application triggers a heartbeat.  XC Alarm monitors the heartbeat events.

# XC Rtu

## XC Rtu DDE Interface

### Introduction

The XC Rtu is responsible for all communications with the RTUs.  The XC Rtu supports automatic polling for archived or current data when is stored in to a real time RAM resident data base.  The data values are then available to other programs such as Wonderware Intouch for display.  XC Rtu also supports many manually initiated operation such as uploading and downloading setups, starting and stopping RTUs, and other SSP operations. Uninitiated messages such as alarms are also supported. Setup information for the stations is stored in the XConnect.INI file and accessed via XC Setup.

XC Rtu has 4 DDE topics:

- Control

- Data

- Auto

- Command

- DataServe

### DDE Information

DDE APPLICATION NAME:   XCRTU

DDE TOPIC NAME:   CONTROL

The following codes are used for the control items, 10, 11 and 12 can be DDE poked to a control item, and 1 through 9 are returned as a status from a DDE request once the operation begins.

| 0 | Initial State, No Data |
| --- | --- |
| 1 | Operation OK |
| 2 | Operation Failed |
| 3 | Disconnecting from RTU |
| 8 | Operation in Progress |
| 9 | Connecting to an RTU |
| 10 | Do an operation, such as connect to an RTU |
| 11 | Disconnect from an RTU |
| 12 | Cancel an opertion. Eliminates all occurrences of an operation from the message queue and if it is in progress, aborts the current operation. |

| ITEM | A | P | R | E | DATA | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
| CONNECT,rtu | x | x | x |  | 1,2,3,9,10,11,12 | Poke it to: Connect or Disconnect from an RTU. Request it to: Check on manual or automatic connect status. All control items will automatically connect before sending a command, so you only need to use this control when you want to make, verify, and hold a connection - such as when performing control loop operation at the RTU. Poking an 11 does not cancel the operation, instead it causes a low priority Disconnect command to be placed in the que. The disconnect will take place after all other other operations have been completed. Poking a 12 can be used to cancel a connect operation. |
| POLL,rtu,CURRENT | x | x | x |  | 1,2,3,8..12 | Request current data from an RTU. |
| POLL,rtu,DATE | x | x | x |  | 1,2,3,8..12 | Request archive data from an RTU based on the LastPoll date stored in the setup. |
| POLL,rtu,NEWEST | x | x | x |  | 1,2,3,8..12 | Request archive data from an RTU based on the date of last poll stored in the RTU. |
| POLL,rtu,MANUAL,startdate,stopdate | x | x | x |  | 1,2,3,8..12 | Request archive data from an RTU from startdate to stopdate. Date format is: MM/DD/YYYY HH:MM:SS |
| POLL,rtu,MANUAL,startdate,stopdate ,mask | x | x | x |  | 1,2,3,8..12 | Request archive data from an RTU from startdate to stopdate, but only those log entries where the record id matches the character mask. (Not supported by the 8200) |
| POLLGROUP,group | x | x | x |  | 1,2,10 | Causes a poll group to be polled immediately. The poll group will also be polled at its regularly scheduled time. Only enabled poll groups can be polled, but they do not have to be in the automatic poll list. The return status only indicates start of polling not completion or final success. To find out whether the poll worked you will have to check the poll status for each station individually (i.e. POLL,rtu,DATE or other option). Reasons for this function failing include the Poll Monitor Window has been shutdown or that the specified poll group does not exist or is not enabled. |
| SENDTAG,rtu,sensor1[:a-b],etc... | x | x | x |  | 1,2,3,8..12 | Send the current value of specified sensors to an RTU. A numeric range can be used to help specify multiple sensor values to send. For instance RainFall:1-3 would be the same as listing out: "RainFall:1,RainFall:2, RainFall:3". |
| GETTAG,rtu,sensor1[:a-b],etc... | x | x | x |  | 1,2,3,8..12 | Requests the current value of specified sensors from an RTU. A numeric range can be used to help specify multiple sensor values to get. |
| SENDSETUP,rtu | x | x | x |  | 1,2,3,8..12 | Sends the setup stored in rtu.$SETUP to the RTU. The format is always 9000.OBJ. This method exists to allow downloading a setup previously placed in memory with GETSETUP (but is rarely used). 8200/8210/9000 only |
| SENDSETUP,rtu,filename | x | x | x |  | 1,2,3,8..12 | Sends the setup stored in filename to the RTU. 8200/8210/9000 only |
| SENDSETUP,rtu,* | x | x | x |  | 1,2,3,8..12 | Sends the setup stored in the default setup file to the RTU. 8200/8210/9000 only |
| GETSETUP,rtu | x | x | x |  | 1,2,3,8..12 | Requests the setup from an RTU and stores it to rtu.$SETUP. The format is always 9000 .OBJ. This exists to allow uploading a setup without creating a disk file (but is rarely used). |
| GETSETUP,rtu,filename | x | x | x |  | 1,2,3,8..12 | Requests the setup from an RTU and stores it to filename. Handles 9000 .OBJ format or 8200 .SET and .BAS format. For an 8200, pass the name of the .SET file, the basic file is assumed to have the same name with an extension of .BAS. 8200/8210/9000 only |
| GETSETUP,rtu,* | x | x | x |  | 1,2,3,8..12 | Requests the setup from an RTU and stores it to the default setup file. For an 8200, the basic portion of the setup will be saved under the same name as the setup file with an extension of .BAS. 8200/8210/9000 only |
| MAIL,rtu[,message] | x | x | x |  | 1,2,3,8..12 | Sends a mail message containing rtu.$OutBox to the RTU. If the optional message is included, then that message is sent instead of the contents of rtu.$OutBox, however it will be sent in upper case. |
| SETCLOCK,rtu | x | x | x |  | 1,2,3,8..12 | Sets the clock of the RTU to the current time of the PC. |
| SELECTCONTROL,rtu,sensor:a-b | x | x | x |  | 1,2,3,8..12 | Loads sensors values in to an RTU, and checks to see if the RTU will accept a BeginControl. SelectControl and BeginControl are used to implement a check back before operate control. 9000 only. |
| BEGINCONTROL,rtu | x | x | x |  | 1,2,3,8..12 | If SelectControl worked then BeginControl is used to eval the sensor to start the operation. 9000 only. |
| SELFTEST,rtu | x | x | x |  | 1,2,3,8..12 | Instructs the RTU to perform a self test. 8200/8210/9000 only. |
| CLEARSTATUS,rtu | x | x | x |  | 1,2,3,8..12 | Clears the system status of the RTU. 8200/8210/9000 only. |
| GETSTATUS,rtu | x | x | x |  | 1,2,3,8..12 | Requests the system status of the RTU. |
| RESET,rtu | x | x | x |  | 1,2,3,8..12 | Causes the RTU to perform a hard reset. 8200/8210/9000 only. |
| ERASESETUP,rtu | x | x | x |  | 1,2,3,8..12 | Causes the setup in the RTU to be erased. 8200/8210/9000 only. |
| START,rtu | x | x | x |  | 1,2,3,8..12 | Start RTU processing and recording. |
| STOP,rtu | x | x | x |  | 1,2,3,8..12 | Stops RTU processing and recording. |
| STARTTAG,rtu,tag | x | x | x |  | 1,2,3,8..12 | Runs the start code of a tag in the 9000. 9000 only. |
| EVALTAG,rtu,tag | x | x | x |  | 1,2,3,8..12 | Runs the eval code of a tag in the 9000, causes a sensor to be measured in the 8200. |
| STOPTAG,rtu,tag | x | x | x |  | 1,2,3,8..12 | Runs the stop code of a tag in the 9000. |
| RESETSETUP | x | x | x |  | 1,2,3,8..12 | Causes the XC Rtu to flush its setup information cache and re-read station information from XC Setup |
| RESETSETUP,ALL | x | x | x |  | 1,2,3,8..12 | Causes the XC Rtu to shutdown and restart all communications monitors and polling, re-reads all setup information. |
| GETBERTSTATUS,rtu | x | x | x |  | 1,2,3,8..12 | Requests the current BERT status of an RTU and places the information in to rtu.$BERTGOOD, rtu.$BERTBAD, and rtu.$BERTDIST. |
| GETBERT,rtu | x | x | x |  | 1,2,3,8..12 | Requests that an RTU send a BERT message. A BERT message is a 1024 byte message consisting of 256 four-byte blocks designed to detect errors in communication links. The results are available by requesting RTUSERVE¦COMMAND!COMINFO,comport,xxx where xxx is BERTGOOD, BERTBAD, and BERTDIST. |
| SENDBERT,rtu | x | x | x |  | 1,2,3,8..12 | Sends a BERT message to an RTU. A BERT message is non-addressable, so other RTUs within range will also be able to here the BERT message. |
| CLEARBERT,rtu | x | x | x |  | 1,2,3,8..12 | Clears the BERT status of an RTU, setting them to 0. |

Operations: A-Advise/UnAdvise, P-Poke, R-Request, E-Execute

#### Message Priority

When SSP messages are dispatched via a scheduled poll or on-demand, they are entered in the message queue of the appropriate Communication Monitor in XC Rtu.  The are executed from the queue based on the following rules:

- Priority

- Station Name (alphabetically)

- Operation code (control command)

Priority Options

Some commands can be scheduled with a specific priority.  To trigger a command add the priority before the item (command): %EPOLL,rtu,CURRENT.

| pri_Emergency | %E |
| --- | --- |
| pri_Override |  |
| pri_High | %H |
| pri_Medium | %M |
| pri_Low | %L |
| pri_AutoPoll |  |
| pri_OnPoll | %O |
| pri_AfterPoll | %A |
| pri_None |  |
| pri_Illegal |  |

Operation Code Priority Options

See detailed description of operation codes above.

qop_SSPConnectRTU

qop_SSPDisconnectRTU

qop_SSPPollCurrent

qop_SSPPollNoWait

qop_SSPPollNewest

qop_SSPPollDate

qop_SSPPollManual

qop_SSPSendSetup

qop_SSPGetSetup

qop_SSPMail

qop_SSPSetClock

qop_SSPSelectControl

qop_SSPBeginControl

qop_SSPSelfTest

qop_SSPClearStatus

qop_SSPGetStatus

qop_SSPReset

qop_SSPEraseSetup

qop_SSPStart

qop_SSPStop

qop_SSPStartTag

qop_SSPStopTag

qop_SSPEvalTag

qop_SSPSendTag

qop_SSPGetTag

qop_SSPBertStatusReq

qop_SSPBertClear

qop_SSPBertReq

qop_SSPBertSend

qop_SSPSendRDI

qop_SSPGetRDI

qop_None

DDE APPLICATION NAME:   XCRTU

DDE TOPIC NAME:   DATA

Sensor alarm values are made up of one or more alarm characters (alarms are the second value in a tag), below is a table of alarm characters:

| ALARM CHAR | DESCRIPTION | ALARM CHAR | DESCRIPTION |
| --- | --- | --- | --- |
| OK | Everything is normal | 1 | Intermediate Low Limit Alarm (9000) |
| H | High Limit Alarm | T | Control Timeout (9000) |
| L | Low Limit Alarm | F | Control Failure (9000) |
| R | Rate Of Change Alarm (8200/8210) | S | Control Software Error (9000) |
| + | Trend Rising (8200/8210) | C | Control Hardware Error (9000) |
| - | Trend Falling (8200/8210) | D | Digital Alarm (9000) |
| A | In Alert | I | Alarm Inhibited (9000) |
| h | Intermediate High Limit Alarm (9000) |  |  |

| ITEM | A | P | R | E | DATA | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
| station.sensor | x | x | x |  | a number, an alarm string, a character, or a boolean "Yes" or "No" depending on the type of sensor. | Current real time value of a particular sensor from a station. Results of a current data poll or a get tag operation will update these values. You must set these values before performing a send tag operation |
| station.sensor :n | x | x | x |  | same as station.sensor | Same as station.sensor but allows for multiple values under the same sensor name. |
| station.$LASTUPDATE | x | x | x |  | mm/dd/yyyy hh:mm:ss | Time and date of last received current data poll. |
| station.$LASTALARM | x | x | x |  | mm/dd/yyyy hh:mm:ss | Time and date of last received alarm. |
| station.$TIMETAG | x | x | x |  | mm/dd/yyyy hh:mm:ss,sensor,data, sensor,data, ... | Last time tag data received consisting of a date and time stamp followed by as many sensor names and data values as exist in the log record received from the station. |
| station.$LASTPOLL | x | x | x | x | mm/dd/yyyy hh:mm:ss | Time and date of last data from a timetag (date) poll. |
| station.$SETUP | x | x | x |  | 9000 SDL object file format | Setup received from a station, or setup to send to a station as used by the SENDSETUP/GETSETUP commands. |
| station.$STATUS | x | x | x |  | number | A 32-bit bit masked number returned by the GETSTATUS command. |
| station.$INBOX | x | x | x |  | string | Last mail message received from the station. |
| station.$OUTBOX | x | x | x |  | string | Mail message to send to the station using the MAIL command. |
| station.$RXCOUNT | x | x | x |  | number | Total number of good messages received from this station addressed to the master station. |
| station.$RXTOTAL | x | x | x |  | number | Total number of good messages received from this station addressed to the master station. |
| station.$RXBAD | x | x | x |  | number | Total number of bad messages from this station, where the from station part of the header was ok but data was lost elsewhere resulting in a CRC error. |
| station.$TXCOUNT | x | x | x |  | number | Total number of messages sent to this station. |
| station.$TXBAD | x | x | x |  | number | Total number of times this station failed to respond to a request for information even after all retries were attempted. |
| station.$POLLSTATUS | x | x | x |  | 0,1,2 | Status of the last POLL control performed on this station. 0 - means no poll has been performed yet 1 - means the last poll was good 2 - means the last poll was bad This status can be more useful than the results of the POLL control itself for detecting communication problems, because the POLL control is forever changing value from 10 down to 1 or 2 as polls cycle over and over. |
| comport.$DECODE | x | x | x |  | to,from,seqnumstatus, opcode,data | Last decoded message from the specified comport (COM1..COM9). This field only updates when the SSP Decoder window is open, although it will update will the screen is paused. |
| station.$BERTGOOD | x | x | x |  | number | Number of good BERT blocks received by this station. There are 256 BERT blocks in a single BERT message. This is set by first polling for BERT statistics using the GETBERTSTATUS control. |
| station.$BERTDIST | x | x | x |  | 32-bit number | Error distribution of blocks in the BERT message. There are 32 bits in this base10 number representing 256 blocks, so each bit represents an error in 8 blocks. The most significant bit represents the first block, so that when the number is displayed in hexadecimal or binary errors in early blocks will show up at the beginning and errors in later blocks show up at the end. A value of 0 would mean that no errors have occured, a value of 4294967295 (FFFFFFFF16) means that errors are distributed in possibly all the blocks. |
| $TIMETAG | x | x | x |  | mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data | Last time tag data received from any station consisting of a date and time stamp followed by pairs of station.sensor names and data values. |
| $REALTIME | x | x | x |  | mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data, | Last alarm or current data received from any station consisting of a date and time stamp followed by pairs of station.sensor names and data values . Current data results from a current data poll, whereas alarm data occurs randomly. |
| $ALARM | x | x | x |  | mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data | Last alarm data received from any station consisting of a date and time stamp followed by pairs of station.sensor names and data values. |
| $CURRENT | x | x | x |  | mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data | Last current data received from any station consisting of a date and time stamp followed by pairs of station.sensor names and data values. |

Operations: A-Advise/UnAdvise, P-Poke, R-Request, E-Execute

DDE APPLICATION NAME:   XCRTU

DDE TOPIC NAME:   AUTO

The AUTO topic allows access to the exact same points as in the DATA topic with the addition that points can be automatically sent to field RTUs when poked, or polled from the RTU when an advise link is created. The AUTOPOLL and AUTOUPDATE parameters (settable in the XCONNECT.INI file under the [RTUServer] topic or changable in real-time via the COMMAND topic), enable and control how often SENDTAG and GETTAG controls are are sent to RTUs. An additional feature of the AUTO topic is that polled data will not overwrite poked data until a SENDTAG control has been generated with the poked data. This ensures that the data you poke is the data that is sent, whereas in the DATA topic if a poll for current data might replace the data you intended to send to the station before you had the chance to issue a SENDTAG control.

| ITEM | A | P | R | E | DATA | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
| station.sensor | x | x | x |  | a number, an alarm string, a character, or a boolean "Yes" or "No" depending on the type of sensor. | Current real time value of a particular sensor from a station. Results of a current data poll or a get tag operation will update these values. Poking a value will initiate a SENDTAG control, advising a data point will initiate GETTAG controls to fetch the current value of a data point. |

Operations: A-Advise/UnAdvise, P-Poke, R-Request, E-Execute

DDE APPLICATION NAME:   XCRTU

DDE TOPIC NAME:   COMMAND

| ITEM | A | P | R | E | DATA | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
| AUTOPOLL |  | x | x |  | number | Determines how often (in seconds) GETTAG controls will be issued to RTUs in order to update hot-linked data points in the AUTO topic. A value of 0.0 disables automatic polling. A negative number will cause a single poll to take place using a delay based on the absolute value, and then AUTOPOLL will be reset to 0. |
| AUTOUPDATE |  | x | x |  | number | Determines how long the XC Rtu will wait before issuing SENDTAG control to update RTUs after a data point in the AUTO topic has been changed. A value of 0.0 disables automatic updating. A negative number will cause a single pol to take place using a delay based on the absolute value, and then AUTOUPDATE will be reset to 0. |
| POLLINFO,GROUP |  |  | x |  | groupnum | Group number of next poll group to be polled. |
| POLLINFO,TIME |  |  | x |  | mm/dd/yyyy hh:mm:ss | Date and time that next poll will take place. |
| POLLINFO,STATUS |  |  | x |  | lines-of-text | Contents of the Poll Group Scheduler's status display list box. |
| COMINFO,comport,STATUS where comport is COM1,COM2,... |  |  | x |  | lines-of-text | Contents of the status list box from the Communication Monitor for the selected comport. |
| COMINFO,comport,ERROR |  |  | x |  | lines-of-text | Contents of the error status list box from the Communication Monitor for the selected comport. |
| COMINFO,comport,MESSAGE |  |  | x |  | lines-of-text | Contents of the message que list box from the Communication Monitor for the selected comport. |
| COMINFO,comport,STATS |  |  | x |  | tx,rx,rxcount,rxtotal, rxbad,txcount,txrepeat | Contents of all statistic boxes from the Communication Monitor for the selected comport'. Each statisitic is described below (this option lets you grab them all at once instead of one at a time). |
| COMINFO,comport,TX |  |  | x |  | "Yes" or "No" | Status of the TX lite from the Communication Monitor for the selected comport'. Returns "Yes" if an SSP message is being sent. |
| COMINFO,comport,RX |  |  | x |  | "Yes" or "No" | Status of the RX lite from the Communication Monitor for the selected comport'. Returns "Yes" if an SSP message is being received. |
| COMINFO,comport,RXCOUNT |  |  | x |  | number | Value of the RXCOUNT box from the Communication Monitor for the selected comport'. This is the number of received messages addressed to the base station. |
| COMINFO,comport,RXTOTAL |  |  | x |  | number | Value of the RXTOTAL box from the Communication Monitor for the selected comport'. This is the number of received messages addressed to anyone. |
| COMINFO,comport,RXBAD |  |  | x |  | number | Value of the RXBAD box from the Communication Monitor for the selected comport'. This is the number of bad messages received. Bad messages occur because of CRC errors, dropped characters, and protocol errors. |
| COMINFO,comport,TXCOUNT |  |  | x |  | number | Value of the TXCOUNT box from the Communication Monitor for the selected comport. This is the number of transmitted messages. |
| COMINFO,comport,TXBAD |  |  | x |  | number | Value of the TXBAD box from the Communication Monitor for the selected comport'. This is the number of transmitted messages which failed because they could not be sent or an acknowledgment was never received. |
| COMINFO,comport,TXREPEAT |  |  | x |  | number | Value of the TXREPEAT box from the Communication Monitor for the selected comport'. This is the number of repeated messages (messages stored and forwarded to another station or com port). |
| COMINFO,comport,BERTGOOD |  |  | x |  | number | Number of good BERT blocks received on this com port. There are 256 BERT blocks in a single BERT message. |
| COMINFO,comport,BERTBAD |  |  | x |  | number | Number of bad BERT blocks received by on this com port. Bad blocks are accumulated by counting the number of missing blocks between two good blocks. |
| COMINFO,comport,BERTDIST |  |  | x |  | 32-bit number | Error distribution of BERT blocks received on this com port. There are 32 bits in this base10 number representing 256 blocks, so each bit represents an error in 8 blocks. The most significant bit represents the first block, so that when the number is displayed in hexadecimal or binary errors in early blocks will show up at the beginning and errors in later blocks show up at the end. A value of 0 would mean that no errors have occured, a value of 4294967295 (FFFFFFFF16) means that errors are distributed in possibly all the blocks. |
| COMINFO,comport,STATS |  | x |  |  | 0 | Poking a zero causes all communication statistics to be cleared for the Communications Monitor on the selected comport. |
| ABORT,comport |  | x | x |  | 0,1 | Poking a 1 will abort all operations performed by the communications monitor for the selected comport. All operations in the message que will be flushed (after the current operation has been aborted). Poking a 0 will clear the abort flag and return operations to normal. Requesting the value will return the status of the abort flag as set by software or manually by the operator: 1=Aborting, 0=Normal. |
| CANCEL,comport |  | x | x |  | 0,1 | Poking a 1 will cancel the current operation(s) in progress by the communications monitor for the selected comport. Poking a 0 will clear the cancel flag and return operations to normal. The cancel flag will automatically return to 0 after the current operation(s) is canceled. The message que is not cleared. Requesting the value will return the status of the cancel flag as set by software: 1=Canceling, 0=Normal. Canceling a communications monitor which is aborted has no effect. |
| PROTOCOLON,comport |  | x | x |  | No,Yes | Poking PROTOCOLON with Yes, On, or 1 turns the protocol analyzer window on for the specified com port - poking No, Off, or 0 turns it off. Requesting returns the current state of the protocol analyzer window: Yes if its on or No if its off. |
| DECODEON,comport |  | x | x |  | No,Yes | Poking DECODEON with Yes, On, or 1 turns the SSP decoder window on for the specified com port - poking No, Off, or 0 turns it off. Requesting returns the current state of the SSP decoder window: Yes if its on or No if its off. |

### Setup Information

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [RTUServer] | MasterID | XCONNECT | SSP Unit ID of your Base Station |
|  | ListenID |  | List of Unit ID's to listen in and parse messages addressed to, but not respond to. This allows one base station to capture messages being sent to one or more base stations without having to poll for the data itself. This is not a perfect method of data collection since the listening base station may drop a message, which the master base station receives ok - and hence does not perform a retry. ex: Master1,Master2,Master3 |
|  | ComPort |  | List of defined com ports. ex: COM1,COM2,COM3 |
|  | PollGroups |  | List of defined poll groups. ex: 1,2,3,4,6 |
|  | Run |  | List of programs to run on start up before the XC Rtu starts to collect data. ex: EXCEL.EXE |
|  | AutoPoll | 0.000 | Determines how often (in seconds) GETTAG controls will be issued to RTUs in order to update hot-linked data points in the AUTO topic. A value of 0.0 disables automatic polling. |
|  | AutoUpdate | 0.000 | Determines how long the XC Rtu will wait before issuing SENDTAG control to update RTUs after a data point in the AUTO topic has been changed. A value of 0.0 disables automatic updating. |
|  | RePollLimit | 00:01:00 | When XC Rtu polls for time tag data from a unit and the response contains decoded sensors which are empty then XC Rtu will only update the time of LastPoll if the time stamp is older than RePollLimit. This handles two conflicting situations ... 1) The RTU may still be collecting data which was empty and a repolling later will collect it, and 2) The entry may have been skipped and all the re-polling in the world may never retrieve a value. Since an RTU usually logs data in “real time” and rarely logs data back in time, RePollLimit should be set to at least the longest logging interval in the system. RePollLimit has a second use in conjunction with the TimeLimit option under [POLLGROUP:n]. When a TimeLimited poll receives no data, the LastPoll date will be automatically advanced anyway until it reaches RePollLimit minutes before the current time. |
|  | ListenUpdate | No | Causes XC Rtu to store the Last Poll date for time tag data that was received thru a ListenID. |

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [COMn] where n=1,2,3...9 | Type | None | Type of device connected to com port, one of: None, Direct, Radio, Modem, or Hub. Radio implies a Sutron Base Station LOS Radio Modem or compatible. Modem implies a Hayes compatible AT-command set telephone modem. Direct implies a direct hookup to an RTU. Hub implies a message concentrator is connected to the port (similar to a direct connect, except logging-in and software handshake are not used). A Hub collects messages from a single port and re-transmits them out multiple ports at the same time. The Hub then collects the responses and ships them back to the base station. This allows for high speed parallel communications to multiple stations at once. See "NoWait" option below. |
|  | Baud | 1200 | Baud rate of device |
|  | DialRetries | 0 | Number of times to re-dial a number if cannot get thru on the first attempt. |
|  | SSPRetries | 2 | Number of times to retry an SSP message if a response is not received. |
|  | NumberOfRings | 2 | Number of rings to answer the phone. |
|  | CarrierDelay | 1.0 | Number of seconds to raise carrier before transmitting data on a radio. |
|  | NullDelay | 0.0 | Number of seconds of null characters to send before sending data. Can be used to allow an RTU to wake-up when using modems without CD. |
|  | UnKeyDelay | 0.0 | Number of seconds to hold carrier after data has been transmitted. |
|  | ReplyDelay | 0.0 | Number of seconds to pause before sending an SSP message. This value is sometimes needed to allow a radio repeater time to settle before responding to a request. |
|  | ConnectTimeout | 30.0 | Number of seconds of inactivity to hold a manually requested connection before hanging up. |
|  | NoConnectTimeout | 60.0 | Number of seconds to wait for the device to become available to attempt a connection. |
|  | DialTimeout | 60.0 | Number of seconds to wait for a phone number to be dialed and the modems to connect. |
|  | AutoTimeout | 1.0 | Number of seconds of inactivity to hold an automatic connection before hanging up. |
|  | AckDelay | 10.0 | Number of seconds to wait for an acknowledgement after a message has been sent. |
|  | ResponseTimeout | 30.0 | Number of seconds to wait for a response once a request has been made, should include slack time for the field unit to timeout and re-transmit a dropped message. |
|  | TouchTone | Yes | Enables tone dialing with a modem, otherwise "No" would cause the modem to pulse dial. |
|  | Handshake | Yes | Enables software handshaking and synchronization before a message is sent. |
|  | HWHandshake | Yes when Type is Modem | Enables RTS/CTS hardware handshaking on the serial port. This is often necessary for high speed telephone modems. |
|  | ModemInitString |  | String to be sent to the modem when the modem is being initialized, do not include the AT. ex: M2L3 would enable the modem speaker all the time and set the volume to the highest level. |
|  | HangUpDelay | 0 | Number of seconds to delay after the telephone modem hangs up/disconnects. |
|  | ModemHangupString |  | String to be sent to the modem when the modem is being hung-up, do not include the AT. ex: M1L2 would put the modem speaker back to normal. |
|  | ControlLine |  | DEFAULT Sets the serial port control line to use for detecting a connection or detecting carrier, should be one of: DEFAULT, NONE, CTS, DSR, or CD. The DEFAULT setting uses NONE for a direct connect or CD for a modem or radio. |
|  | NoWait | No | Setting NoWait to Yes will allow SSP messages to be sent to the device while other messages are still being processed. However, simultaneous messages to the same station will still remain in the message queue to prevent confusing an RTU which can only handle one conversation at a time. This option is meant to be used when the com port is connected to a Hub, and allows the Hub to operate on more than one message at a time. If NoWait is enabled on a modem or radio link, collisions will occur resulting in decreased throughput and transmission timeouts. |
|  | AutoBaud | Yes | Causes the baud rate of the communications port to be switched to match the "CONNECT xxxx" message returned by a modem after dialing and connecting. Some modems (like older Microcom models) report one rate but stay at the original rate and should have this option set to No. |
|  | ConnectDelay | 0.0 | Number of seconds to pause after connecting to the device before sending characters. Some combinations of modems & modem protocols may require a few seconds of delay before characters are sent after dialing-out and connecting. When characters are sent to soon sometimes all following characters are dropped and on some modems a disconnect may occur. |
|  | MaxParsers | 16 | Controls the maximum number of messages which will be issued at the same time when using NoWait=Yes on a Hub based system. Performance bogs down when too many parsers are running monitoring the performance of multiple messages. This options allows you to "throttle" back the system so too many messages are not dispatched. As a rule of thumb, a setting of 8 is appropriate for a 386/33 level machine, 16 is good for a 486/33 machine. You can tell if the Base Station is overloaded by monitoring whether it responds to incoming messages within a few seconds. |
|  | TXPort | 600 | For Virtual ports using IP address, port number of socket to transmit SSP messages. |
|  | RXPort | 500 | For Virtual ports using IP address, port number of socket to receive SSP messages. |
|  | Protoco | UDP | For Virtual ports using IP address, transport protocol to use. |

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [POLLGROUP:n] where n=1,2,3,4... | Enable | No | Set to "Yes" to enable polling for this group. |
|  | BaseTime | 00:00:00 | Offset from the top of the hour to start polling data from the group. |
|  | Interval | 00:00:00 | How often to poll the group. |
|  | Stations |  | List of stations to poll and/or send time syncs to in this group. |
|  | Clients |  | List of applications which must be running before this group will be polled. The names must match the window title of the application which is usually not the same as the executable file name or the DDE server name. ex: XC Export |
|  | TimeSync | No | Set to "Yes" to cause a SETCLOCK message and a poll request to be sent to each station. Set to "Only" to to cause only the SETCLOCK message to be sent. A SETCLOCK message will correct the RTU's clock with the current time in the master station PCs. 8200's will update right away, while 9000's must perform a timesync operation before they will update. |
|  | PollType | None | When set to something other than None, this option will override the default PollType assigned to each station. For more information on valid values see PollType under the [Station] topic below. |
|  | TimeLimit | 00:00:00 | If set to a value other than 00:00:00, time limit will limit the amount of data that will be collected by a poll by date. This is very useful in preventing a station which has gotten very far behind in polling from hogging the communications port. ex: 08:00:00 would limit date polls to collecting no more than 8 hours of data at one time. The actual time required to collect 8 hours worth of data will vary depending on the communications rate and the logging interval of the station. Sometimes when a field station has been down it may contain no data for the poll’ed time period. XC Rtu will automatically advance the date of LastPoll in this case until the polling catches up with real time. The time that determines when XC Rtu stops automatically advancing is determined by RePollLimit in the [RTUServer] topic. |
|  | Control |  | Allows the actual control performed on each station in the poll group to be defined explicitly, so that pollgroups which perform actions other than archive or current data polls can be performed. Any control topic item can be used. Use "%p" to indicate where the station name should appear in the control if needed. ex: GETSETUP,%p,* would cause the pollgroup to perform the GETSETUP control on each station in the stations list. |

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [STATION:station] where station is the name of the station being defined. | Enable | No | Set to "Yes" to enable station for decoding. |
|  | SiteType |  | Type of station being defined, can be one of: 8200, 8210, 9000, 9210, XPERT. |
|  | PollType |  | Determines how the unit will be polled, can be one of the following: DATE, NEWEST, CURRENT, NOWAIT. DATE collects archived data starting after the LastPoll date up until now. NEWEST collects archived data not previously sent as remembered by the RTU. CURRENT collects last measured values only. NOWAIT peforms a current data poll, but does not wait for the response. This is meant to be used when polling through a Hub which will buffer the polls send them out to the field stations simultaneously, and then send back the responses as they arrive. Do not use the NOWAIT option on anything other than a Hub, or massive message congestion will occur. |
|  | ComPort |  | List of com ports which the station can be reached through in the order they should be used. ex: COM1,COM2 would try COM1 first and if it failed would then try COM2. |
|  | RadioPath |  | List of SSP paths for reaching the station, one for each ComPort defined. In the simplest form a SSP path is just the UnitID as defined above, in a more complex system it can include store and forward repeater paths. For instance in a marginal radio system we may first try to reach RTU01 directly, but if that fails, we may try to route the message through RTU02 which is closer first and have it relay the message to RTU01 as shown below: ex: RTU01,RTU02_RTU01 the "_" underscore characters seperates stations in a repeater path. |
|  | AckDelay | defaults to the value specified under the [COMn] topic. | List of acknowledgement timeout values in seconds for each RadioPath as defined above. Each AckDelay value defines how long the base station will wait for a response to a request before timing out and retrying. The longer the path to the RTU or the slower the link the longer delay you will need. Can be as short as 2 seconds for a direct connect, 5 to 15 seconds for a radio or modem, and a multiple of that for each store and forward repeater involved. ex: 10,15 |
|  | ResponseTimeout | defaults to the value specified under the [COMn] topic. | List of response timeout values in seconds for each RadioPath as defined above. Each ResponseTimeout value defines how long the base station will wait for a response to a request before timing out and retrying. The longer the path to the RTU or the slower the link the longer delay you will need. The ResponseTimeout is used in cases where the RTU is acting like a master in the respect that the RTU is sending data and expecting an ack back from the PC. The RTU will automatically retry according to the Ack Delay and SSP Retries set in the RTU. The ResponseTimeout must allow time for the RTU to attempt all of its retries, which can be determined by: AckDelay * (SSPRetries + 1). Can be as short as 15 seconds for a direct connect, 30 to 60 seconds for a radio or modem, and a multiple of that for each store and forward repeater involved. ex: 30,60 |
|  | UnitID |  | The Unit ID or Name of the station as defined at the station - can be different than the station name. ex: RTU01 |
|  | SetupFile |  | Default filename for storing and retrieving setups for this station. ex: C:\BASE\SETUPS\RTU01.SET |
|  | PhoneNumber PhoneNumber2 |  | The primary and secondary phone numbers to dial to reach the station if it is connected to a modem. You can include any extra characters which your modem understands. For instance adding a comma to the phone number will cause a short delay in dialing. ex: 1-800-555-1212 |
|  | UserName |  | This is the user name you wish the base station to use when logging in, for 8200's the UserName is the same as the UnitID when logging in is enabled. We recommend setting up user names and passwords in your RTUs, not only for security reasons, but because it will help the base station login more reliably. |
|  | Password |  | This is the password the base station will need to use to login, for 8200's the Password will be blank if an external modem is used; otherwise it is defined in the modem setup menu. |
|  | LoginReqd | No | Tells the base station whether it will need to login with a UserName and Password to access the station. |
|  | AutoProto | No | Set this to "Yes" when a modem or direct connection to the RTU is always in SSP protocol mode and login is not required. |
|  | QuickProto | Yes | All recent 8200's and 9000's support this feature which allows the base station to switch directly to SSP protocol mode when logging in by placing a "/C" after the user name. This allows the base station to avoid having to traverse the RTU menus and select the "A"-Application Menu for entering protocol mode. |
|  | Sensors |  | List of sensor names defined for the station, does not have to match the names used in the RTU. ex: Air Temp,WindSpeed,WindDir,Battery |
|  | LastPoll | 12/31/1984 00:00:00 | Stores the date and time of the last poll as used when polling the station for archived data by date. You can move the date back to cause old data to be repolled, or set it ahead to cause data to be skipped. |
|  | Concentrate | No | When set to Yes, tells XC Rtu that the station is a concentrator of other station’s data in its log. If the first item in a log record from a concentrator is the unit id of a station in the setup then the log record will be interpreted as if it had come from that unit. The record id will be switched to 0, which is the default for 8200’s. If the first item does not match any existing units, then it will be processed normally as a record id belonging to the station. Currently the only device which supports concentration is the 9000 RTU. With special SDL software 9000’s can poll 8200’s for current data and concentrate this information in to the 9000’s log. 9000’s can only concentrate current data, because they do not support polling for time tag data. A better method of concentration is to use multiple base stations, and share data with Poll Data. This is not always possible if the concentrator needs to run off batteries or at remote locations. |
|  | HoursAhead | 0 | Set this to the number of hours the RTU’s time is ahead of the local time at the base station. Use a negative number to specify the number of hours the RTU is behind the base station. This amount will be added to outgoing and subtracted from incoming timestamp’s in messages processed by XC Rtu. For GOES messages there is a single GMT offset and HoursAhead is not used. ex: (Base station is EST and RTU is PST) HoursAhead=-3 ex: (Base station is EST and RTU is GMT) HoursAhead=6 |
|  | RDIBaseTime | 00:00:00 | Used for EDACs systems only, offset from the top of the hour when the 8210/XPERT will turn on the EDACs radio. |
|  | RDIInterval | 00:00:00 | Used for EDACs systems only, how often the 8210/XPERT will turn on the EDACs radio. |
|  | RDIDwell | 00:00:00 | Used for EDACs systems only, how long the 8210/XPERT will leave the EDACS radio turned on. |

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [UNITS] | unitid=station |  | Each station defined in the setup must have an entry in the UNITS topic to allow translation from a UnitID to a Station Name. The UnitID is the name of the RTU as set in the RTU, the Station Name is the name of the RTU as set in the base station. If they are the same name, then no entry is necessary. ex: RTU01=Potomac River Stage |

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [TAGINFO:station] | tagname=sensor |  | Each sensor defined in the a station setup must have an entry in the TAGINFO:station topic to allow translation from a tag name to a sensor name. The tag name is the name of the parameter as set in the RTU, the sensor name is the name as set in the base station. If they are the same name, then no entry is necessary. ex: Analog1=Air Temperature |

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [LOGINFO:station] | recordid=sensor1,sensor2,sensor3... |  | Each sensor you wish to collect archived data for should be listed in a LOGINFO:station topic. The record id identifies the type of log record which are transmitted by the station (always "0" for an 8200), and contains a list of sensor names in the order they appear in the log. For the 9000 the record id is the first item in a log entry and must be either a unique character or string. For instance if you had an 8200 named MET5 which was logging 3 parameters: Air Temp, Wind Speed, and Wind Dir in that order, you would need the following entry: [LOGINGO:MET5] 0=Air Temp,Wind Speed,Wind Dir |

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [SENSOR:station:sensor] | Enable | No | Set to "Yes" to enable sensor for decoding. |
|  | TagName |  | This is the name of the sensor as defined inside the RTU. An optional ":value#" can be placed after the tag name to select a different tag value then the default of 1. This information is used for processing send tag and get tag requests. For the 9000 the following values are defined (by convention): :1 - Most recent data for a tag :2 - Alarm Status for a tag :3-n - User defined values For the 8200 the following values are defined: :1 - Most recent data for a sensor :2 - Alarm Status for a sensor :3 - Slope of the sensor :4 - Offset of the sensor :5 - Right digits for the sensor :6 - Measurment counter note: XC Rtu swaps values 2 & 6 as received from an 8200 so that the alarm status will be stored in value 2 for consistency with the 9000. ex: WindSpeed1 for the current reading of WindSpeed ex: WindSpeed1:2 for the alarm status of WindSpeed |
|  | Equation |  | Contains an equation expressed in terms of "X" which will be applied to incoming sensor data before it is stored. The expression is not case sensitive. Applies to sensor data from current data polls, archived data polls, and get tag requests. Uses the syntax of the BASIC language. The following functions are available: ARCTAN, COS, EXP, LOG, LN, SIN, SQRT, TAN, ABS, ROUND, TRUNC, INT and RND. INT(X) returns the integer part of X where as TRUNC and ROUND truncate and round the value. RND(X) returns a random number from 0 to X-1. To raise a number to a power use the "^" (X^2 would square X). Comparison can be performed using <, >, <=, >=, <>, and =. The result of a comparison is 1 for true or 0 for false. The following bitwise boolean operators are supported: AND, OR, XOR, SHL, and SHR. The last two are shift-left and shift-right. For instance (X SHL 4) would shift X left by 4 bits. AND & OR can also be used in logical expressions. For instance “(X>100) OR (X<50)” would result in 1 if X is above 100 or below 50 - otherwise 0. The NOT operator is logical not bitwise. This means that NOT 0 is 1 and NOT 1 is 0. Also, the NOT of any non-zero number is 0. An additional function called TABLE exists which can perform a table lookup using an external file of data points. The syntax for a table lookup is: TABLE(tablefile,expression) where tablefile is the filename that contains the data points, and expression is any valid equation or just "X". See the section on Lookup Table Format after this section for information on how to build a lookup table file. You can also check to see whether a real time value is defined with “?”. For instance ?TEST has a value of 1 if test is defined and 0 if not. Equation (examples) ex: (X AND 128) <> 0 results in a 1 if bit 7 in X is set or 0 if bit 7 is clear. The bit mask 128 is 2^7. This assumes bit 0 is the least signicant bit. In general, the bit mask for any bit N is 2^N. Equations can also contain references to real time values: ex: (X+RTU01.AirTemp)/2 would average an incoming air temperature with its most recent value. ex: X*9/5+32 would convert readings from oC to oF. ex: SIN(X)+COS(X)+X^3+LOG(X) example with multiple functions. ex: TABLE(MYTAB,X) translates X based on a lookup table contained in MYTAB.TBL in the directory specified by TablePath. ex: (X>1000)*1000 + (X<=1000)*X would limit the value so that it could never be greater than 1000. |
|  | RightDigits | 0 | Number of decimal digits to be used when displaying the sensor value. |
|  | Raw |  | If non-blank this is a list of additional sensors to process the raw data from this sensor through. Besides allowing the benefit of additional processing, Raw can also be used to store the same data in to multiple data files. ex: TempF,TempK would cause raw data to be processed by sensors named TempF, and TempK. |
|  | RoundOffTime | 00:00:00 | Determines the interval at which time stamps are rounded off to. For instance with the round off time is 15 minute : 12:48 would be rounded to 12:45 and 12:53 would be rounded to 13:00. |

DDE APPLICATION NAME:   XCRTU

DDE TOPIC NAME:   DATASERVE

The DataServe topic is only valid when using PCBASE2 binary data files as your data storage.

| ITEM | A | P | R | E | DATA | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
| POINT,station.sensor,date |  |  | x |  | number | Retrieves or stores a number to/from for a given station and sensor at a particular date and time. The data file the sensor is stored in is determined automatically from the station and sensor. ex: POINT,Site5.AirTemp,Now would store or retrieve the current value stored for sensor AirTemp from station Site5. |
| BLOCK,datafile,startdate, stopdate[,interval][,sensorlist] |  |  | x |  | r1c1,r1c2<cr><lf> r2c1,r2c2<cr><lf> r3c1,r3c2<cr><lf> | Requests data from a data file from start date to stop date and returns a table of comma (if csv format requested) or tab (if text format requested) seperarated columns and <cr> <lf> delimeted rows of ascii numbers. The interval can be specified to limit the request to only certain time intervals in the data file, and the sensor list can be specified to request data for only certain sensors in the data file. If the sensor list is specified then the interval must also be specified - although an interval of 00:00:00 can be used to select all data. ex: BLOCK,METDAT,Yesterday,Now,01:00:00,Site5.AirTemp would request hourly air temperature data for station Site5 from the data file METDAT from yesterday until now. |
| SENSOR,datafile |  |  | x |  | station.sensor1,station.sensor2, station.sensor(n) | Returns a list of all sensors in a data file including the station the sensors are from. |
| INFO,datafile |  |  | x |  | "desc",startdate,enddate,basetime, interval,use24hr,numsensors | Returns information about a data file including: the data files description, the start date of the file, the stop date and time, the base time, the interval, the Use24Hour flag, and the number of sensors count. |
| FILES |  |  | x |  | filelist | Returns a list of data file names currently supported by the data server. |
| CREATE,datafile,"desc",startdate,basetime,interval, use24hr,station.sensor1,rd1,--station.sensor2,rd2,... |  |  | x |  | 1 - OK 2 - FAILED 3 - Already Exists | Attempts to create a data file and returns either 1, 2, 3 to indicate the status as listed to the left. You must pass the name of the data file to create, a description of the file (inside double quotes), a start date for the file (no time - assumed 00:00:00), a base time (normally 00:00:00 to sync timestamps with the top of the hour), an interval which should be greater than or equal to the smallest time duration you expect in your data stream between data points, the use24hr flag which determines whether days start at 00:00:00 "No" or end at 24:00:00 "Yes" , and finally a list of station.sensors and right digits you wish to store in the data file. The number of right digits for each sensor is used to determine the number of decimal digits used to display data from that sensor. The use24hr flag has an addition effect in that when set to "Yes" time stamps are rounded up to the next higher time stamp, and when set to "No" time stamps are rounded down the the next lower time stamp. For instance if a data file stores hourly data, a piece of data with a time stamp of 1:15 would be stored in the 2:00 slot if the use24hr flag is "Yes" or in the 1:00 slot if the use24hr flag is "No". This is due to the difference in what data belongs in which day according to the different schemes and how the RTUs time stamp their data. ex: CREATE,METDAT,"Meteorological Data",This Year,00:00:00,01:00:00,No,Site5.AirTemp,2,Site5.BaroPress,1 would create a data file called METDAT with a description of "Meterological Data", would allocate room for data starting at the beggining of the current year, with a base time of 00:00:00, a data interval of 1 hour, use24hr off, and with sensors from station Site5 called AirTemp and BaroPress with right digits of 2 and 1. |
| DELETE,datafile |  |  | x |  | 1 - OK 2 - Does not exist | Deletes a data file and returns a result code. |
| SPLIT,datafile,date |  |  | x |  | 1 - OK 2 - FAILED | Splits a data file at the specified date in to two files - the original file will contain data on and after the date, and a new file with the extension ".OLD" (instead of ".DAT") will be created which contains data before the date. This is a useful function for archiving old data, and keeping the data files trimmed down. ex: SPLIT,METDATA,This Month-6/0/0 would keep the last 6 months worth of data plus the current month in the data file and place the rest in "METDATA.OLD". |
| ADJUST,datafile,date |  |  | x |  | 1 - OK 2 - FAILED | Adjusts the start date of a data file to the new data as passed. ex: ADJUST,METDATA,This Month would throw away all data in the data file METDATA before the start of this month, or if the data file previously started after the start of the month will allow data to be stored earlier. |
| CHANGE,datafile,"desc",startdate,basetime, interval,use24hr,station.sensor1,rd1,--station.sensor2,rd2,... |  |  | x |  | 1 - OK 2 - FAILED | Attempts to change or create a data file and returns either 1, or 2 to indicate the status as listed to the left. Documentation for each parameter is discussed on the CREATE command. Change will retain all sensors which have the same name in the new file as in the old. If the interval is increased then some data will be lost. If the interval is decreased then existing data will be replicated to fill in the holes. Changing the use24hr flag may result in data being offset by one time interval. |
| ACTIVATE,datafile |  |  | x |  | 1 - OK 2 - FAILED 3 - Already Exists | Adds an existing data file in the directory specified by the data file path to the Data Server. The filename will be automatically added to the setup so that the file will be opened the next time Data Server is started. This can be used to add a file which has been copied from another machine. |
| DEACTIVATE,datafile |  |  | x |  | 1 - OK 2 - Does not exist | Removes a data file from control of the Data Server, but does not delete the file or any of its data. The file can be added back later with the activate command. |
| EXIST,station.sensor |  |  | x |  | data file name |  |

Operations: A-Advise/UnAdvise, P-Poke, R-Request, E-Execute

## XC Rtu COM Interface

### Introduction

The XC Rtu is responsible for all communications with the RTUs.  The XC Rtu supports automatic polling for archived or current data when is stored in to a real time RAM resident data base.  The data values are then available to other programs such as Wonderware Intouch for display.  XC Rtu also supports many manually initiated operation such as uploading and downloading setups, starting and stopping RTUs, and other SSP operations. Uninitiated messages such as alarms are also supported. Setup information for the XC Rtu is stored in the XConnect.INI file and accessed via XC Setup.

XC Rtu supports a variety of methods and events through its interface.  The XC Rtu interface is named XCRtuCOM.  The methods and events are described below. The methods and events are described below.  Most methods pass a group of parameters, a record, as a variant.  The order of the parameters in the table indicate is the order of the variant record that is passed.  Others functions will pass values as strings

| Type | Field Size |
| --- | --- |
| Boolean | 1 byte |
| Word | 2 bytes (unsigned) |
| Integer | 4 bytes (signed) |
| Longint | 4 bytes (signed) |
| Enumerated | 1 byte |
| Single | 4 bytes |
| Double | 8 bytes |

### XCRtuDataCOM Methods

- CURRENT

- REALTIME

- ALARM

- TIMETAG

- LASTUPDATE

- QUALITY

- ResetTables

- ResetAllParams

- AppShutdown

- SetItemLink

- SetItemChange

- SingleItem

- IsAlive

### XCRtuDataCOM Events

- OnAppShutdown

- OnItemChange

- OnHeartBeat

### Methods

#### Current(Value)

This function allows the user to retrieve or set last data received from an Current data SSP message.  This returns the same data as $CURRENT DDE point.  The data return has the form of:

mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data, station.sensor,data...

#### Realtime(Value)

This function allows the user to retrieve or set last data received from an Current Data or Alarm SSP message.  This returns the same data as $REALTIME DDE point.  The data return has the form of:

mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data, station.sensor,data...

#### Alarm(Value)

This function allows the user to retrieve or set last data received from an Alarm SSP message.  This returns the same data as $ALARM DDE point.  The data return has the form of:

mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data, station.sensor,data...

#### Timetag(Value)

This function allows the user to retrieve or set last data received from an Manual or Date data SSP message.  This returns the same data as $TIMETAG DDE point.  The data return has the form of:

mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data, station.sensor,data...

#### LastUpdate(Value)

This function allows the user to retrieve or set last timestamp of data received from an Current Data or Alarm SSP message.  This returns the same data as $LASTUPDATE DDE point.  The data return has the form of:

mm/dd/yyyy hh:mm:ss

#### Quality(Value)

This function allows the user to retrieve or set last data received from an Current Data or Alarm SSP message.  This returns the same data as $Realtime DDE point.  The data return has the form of:

07/20/2006,12:22:34,station.$PORT,COM1,station.$RXTX,RX,station.$SEQNUM,122223,station.$OPCODE,0,station.$STATUS,0,station.$RETRIES,0,station.$GOOD,0,station.$BAD,0,station.$CRC,0

See SSP Operation Codes for all opcode parameters.

#### ResetTables

This procedures notifies XC Rtu to clear any rating tables it has in memory and re-load rating tables when used.

#### ResetAllParams(AppGUIDStr)

This procedure allows the user to issues a software reset to all XConnect applications that retrieve information from XC Setup.  One parameter is required for the procedure call:  Application GUID string.  For the XConnect applications, when a ResetAllParams command is received, they will stop all processing, close any com ports and the re-request the latest setup information from XC Setup.

#### AppShutdown(AppGUIDStr)

This procedure will broadcast an application shutdown command to XConnect applications.  One parameter is required for the procedure call:  Application GUID string.  This method All XConnect servers and processing applications will respond to this command.  Applications that strictly connect to the database such as XC DataView, XC Reports or XC Export will not.

#### SetItemLink(AppGUIDStr, Item)

This procedure allows a client application to set a link to a specific data item.  XC Setup will send out an event to all clients via their GUID string with the new value.  Two parameters are required for this procedure call:  Application GUID string and item.  An item could be a specific sensor value such as XPERT.BATTERY.

#### SetItemChange(AppGUIDStr, Item, Data)

This procedure allows an application to broadcast an message for a data item value changed.  This provides data to any client applications with hot-links to data items.  Three parameters are required for the procedure call:  Application GUID string, Item, Data.

#### SingleItem(Item, Data)

This function allows the user to retrieve or set a single value from the internal real-time database.  Two parameters are required for the function call: Topic and Item.  The Topic is the parameter in the INI file surrounded by brackets such as [General].  The Item is the parameter entry in the topic, such as Use24Hour.  As a note, station, sensor and poll group information should not be deleted.  This information is stored in the database and will overwrite and user changes.

#### IsAlive(TimeString)

This procedure broadcasts a call with the current date time string (yyyy/mm/dd hh:mm:ss) to indicate the application is functioning properly and not frozen.

### Events

#### OnAppShutdown(AppGUIDStr)

This event notifies XConnect client applications when XC Setup will shutdown.  Many applications such as XC Rtu, XC Decode, XC Alarm or XC Calc require XC Setup, thus when XC Setup shuts down, they will also shutdown.

#### OnItemChange(AppGUIDStr, Item, Data)

This event notifies XConnect client applications when a data item has changed.  This provides data to any client applications with hot-links to data items.  Three parameters are passed with the event:  Application GUID string, Item, Data.

#### OnHeartBeat(AppGUIDStr, TimeString)

This event notifies XConnect applications of the application heartbeat.  Each application triggers a heartbeat.  XC Alarm monitors the heartbeat events.

# XC Mux

## XC Mux DDE Interface

### Introduction

The XC Mux program polls for data from the Sutron receive site Mux hardware and outputs NES/DAPS format files. Alternatively the captured output of the Mux h/w can also be read in from disk by the Mux program and converted to DAPS format. The DAPS files may then be processed by the XC Decode program so that the actual sensor data can be extracted. The XC Mux software displays a status screen, the contents of the last message processed, the current output file, a button for enabling the display of all data in the status screen (usually only error and status messages are display), and a button for displaying the raw communications with the hardware. Menu selections allow captured mux files to read-in and processed, resetting the mux h/w, and setting the clock in the mux (the mux clock is automatically checked once an hour). Menu options also exist for copying the following information to the clipboard and for creating a data link for capturing the information in logger for example: current status screen, current NES message, current mux message, last mux message with errors. The software also supports the installation of a true time clock connected to the mux. When a true time clock is available, the software will automatically update the PC’s time to the true time; when not available the mux time is corrected to match the PC’s time. The mux software can also be shut down for short periods of time with out missing messages, since the mux hardware is capable of buffering several messages. All messages received by the software are output, no filtering is performed.

### DDE Information

DDE APPLICATION NAME:   XCMUX

DDE TOPIC NAME:   DATA

| ITEM | A | P | R | E | DATA | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
| $BAD_DATA | x | x | x |  | text | Line received from the Mux which could not be interpreted or contained only partial data. |
| $MUX_DATA | x | x | x |  | text | Line of data received from the Mux in the Mux’s format. Although the mux sends multiple lines for one message, they have all been concatenated for this DDE point. |
| $DEMOD | x | x | x |  | number | Number of the demod that the last message was received on. |
| LAST_DATE | x | x | x |  | mm/dd/yyyy hh:mm:ss | Time and date of the last message |
| $CHANNEL | x | x | x |  | number | Channel number of the last message. |
| $SATID | x | x | x |  | 8-digit-hex-number | Satellite (Platform) ID of the last message. |
| $DATA_QUALITY | x | x | x |  | text | Data quality of last message: Good, Short Msg, Bad Chan, Parity Err, or ?. |
| $NES_DATA | x | x | x |  | text | Last message received converted to NES data format. See appendix A for more information on this format. |
| $FREQUENCY_ERROR | x | x | x |  | integer | Frequency error of last message (Hz). |
| $SIGNAL_STRENGTH | x | x | x |  | integer | Signal strength of last message (dB). |
| $MODULATION | x | x | x |  | integer | Modulation or phase deviation of last message (degrees). |
| $SIGNAL_NOISE | x | x | x |  | integer | Signal to noise ratio of last message. |
| $PARITY | x | x | x |  | integer | Number of parity errors in last message. |
| $OUTPUT | x | x | x |  | text | Allows manual commands to be sent to the Mux. When non-blank the software will send this string to the Mux. After being sent the field will be set back to blank. |
| $INPUT | x | x | x |  | text | Allows manual queries to be sent to the Mux. When non-blank the software will send this string up to 3 times to the Mux, and wait for a response. After the command is complete the field will be set back to blank. The item $RESULT will be set to the string returned from the Mux. |
| $RESULT | x | x | x |  | text | Contains the result of the last manual query or blank if the last query failed. |
| demod | x | x | x |  | channel,sattelite | demod can take on values ranging from ‘0’ to ‘9’ and ‘A’ to ‘F’. Poking a demod will change the channel and satellite stamped on messages received from that demod. The demod is not re-programmed, that must be done by poking to $OUTPUT. The value will be set to the default contained in the setup initially or after a reset. |
| $RESET | x | x | x |  | list-of-commands | List of strings to be sent to the Mux when reset. Poking will cause the string(s) to be sent to the Mux. On startup the value is set according to the Reset option under the [Mux] setup section. This allows the Mux (and Demods) to be initialized when the s/w is started to a known state, and then changed on the fly to another state. If the user performs a reset mux command, then the Mux (and Demods) will be automatically re-initialized to the new setting. |
| STATUS | x | x | x |  | lines-of-text | Returns the contents of the Mux status display. |

Operations: A-Advise/UnAdvise, P-Poke, R-Request, E-Execute

### Setup Information

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [MUX] | Run |  | List of programs to run when the Mux program is started and before it starts to process data. |
|  | ComPort |  | Com port that the mux is connect to the PC on, COM1 thru COM9. |
|  | Baud | 9600 | Baud rate of com port connected to DRGS. 9600 is default for 100-Baud DDRGS, 19200 is default for HDR DDRGS. NOT USED FOR DSR DRGS. |
|  | DDRGSType | 100-BPS DDRGS | Type of DRGS XC Mux is communicating with. AVailable DRGS types are: '100-BPS DDRGS', 'HDR DDRGS', 'DSR DRGS' |
|  | RawPath | same as used in the RawPath entry in [General] | Directory to store raw NES files in to as messages are received from the mux. |
|  | RawFile | yyyymmdd.RAW | Determines the file to append the raw NES files in to as messages are received from the mux. The lower case letters “yy”, “yyyy”, “mm”, “dd” “hh”, “mi” can be included in the name to instruct the software to create separate minute, hourly, daily, monthly, or yearly files. Optionally “#nn” can be added to specify that a new file should be created every nn minutes (when using “mi”), hours (when using “hh”), or days (all other). “#nn” can appear anywhere in the string, where nn must be two digits with leading zero (if necessary). The unit of time is determined by the smallest time specified. The dating of the raw files is important for a couple of reasons, easy backup, purging, and distribution of raw data; and re-processing of raw data thru the decoder. If the files are allowed to grow too large these processes will become more difficult or require excessive time to perform. The default setting yyyymmdd.RAW creates a separate file for each and every day. ex: MYDATdd.RAW would create the file MYDAT01.RAW on the first day of the month, MYDAT02.RAW on the second day, and so on until the next month when MYDAT01 would be used again. If you did not delete MYDAT01 then it would be appended to and continue to grow. This method might be desirable if you wanted to make sure that no more than 31 files would be created. ex: NESmm.RAW would create the file NES01.RAW to contain all the data for January, NES02.RAW to contain all the data for February, and so on until the next year. This method might be preferable if your system is not storing high quantities of data. ex: yymmddhh.RAW#02 would create a new output file every 2 hours. If the “#02” was left off the, end then a new file would be created every hour. ex: mmddhhmi.RAW#15 would create a new output file every 15 minutes. Since DOS file names are limited to 8 characters the year had to be left out to make room for minutes. |
|  | Reset |  | List of commands to be sent to the Mux (or Demods) upon reset. Also sets the initial value for the DDE field $RESET. ex: P5MC,P5C1231 would set demod 5 to computer mode, and set its channel to 123. NOT USED WITH DSR DRGS TYPE. |
|  | Listen | No | When set to Yes, Xc Mux will act as a listen and send no commands to the DRGS. It is expected there is another PC also running XC Mux that is communicating with the DRGS. |

## XC Mux COM Interface

### Introduction

Oops, our apologies...topic still under construction.

# XC Decode

## XC Decode DDE Interface

### Introduction

XC Decode is a program which automatically or manually reads in ASCII files of NESDIS formatted or Sutron extended raw satellite data messages and extracts sensor values. As the values are decoded they are made available in XC Decode's database for realtime access or logging to disk. Setting up the XC Decode includes describing which stations map to which satellite ID's, how to decode a particular channel, and how to locate and extract sensor values from a message. XC Decode automatically stores the position at which it completed processing a file in to a 10 byte file of the same name but with an extension of ".POS". The ".POS" file is used so that the XC Decode can detect whether futher data has been appended to a message file so it can continue where it left off. To cause the XC Decode to re-process a file from the beginning you must delete the related ".POS" file first.

XC Decode has one DDe topic: Data.

### DDE Information

DDE APPLICATION NAME:   XCDECODE

DDE TOPIC NAME:   DATA

| ITEM | A | P | R | E | DATA | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
| station.sensor | x | x | x |  | a number, an alarm string, a character, or a boolean "Yes" or "No" depending on the type of sensor. | Most recent value of a particular sensor from a station. Data decoded from both selftimed or random transmission are considered most recent as long as the time the data was received is later than the previous data. |
| station.sensor :n | x | x | x |  | same as station.sensor | Same as station.sensor but allows for multiple values under the same sensor name. |
| station.sensor.$LASTUPDATE | x | x | x |  | mm/dd/yyyy hh:mm:ss | Time and date of most recent data received for a sensor from a station. This field is used as the basis of comparison to determine whether a newly decoded piece of data is more recent than a previous value. |
| station.$LASTUPDATE | x | x | x |  | mm/dd/yyyy hh:mm:ss | Time and date of most recent data received for a station. |
| station.$TIMETAG | x | x | x |  | mm/dd/yyyy hh:mm:ss,sensor,data, sensor,data, ... | Last time tag data received from a station consisting of a date and time stamp followed by pairs of sensor names and data values collected at that time. |
| station.$decodetype | x | x | x |  | mm/dd/yyyy hh:mm:ss,sensor,data, sensor,data, ... | Same format as station.$TIMETAG except only contains data received from the station which was decoded using decodetype. For instance station.$GOESRANDOM would contain the last random mesage from “station”. |
| $TIMETAG | x | x | x |  | mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data, ... | Last time tag data decoded from any station consisting of a date and time stamp followed by pairs of station.sensor names and data values. |
| $ decodetype | x | x | x |  | mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data, ... | Same format as $TIMETAG except only contains data which was decoded using decodetype. For instance $GOESRANDOM would contain the last random message from any station. |
| $REALTIME | x | x | x |  | mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data, ... | Most recent data decoded from any station consisting of a date and time stamp followed by pairs of station.sensor names and data values. Data is considered most recent if the time stamp of the data is later than previous data received for a sensor. |
| $QUALITY | x |  | x |  | mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data, ... | All XC Decode quality messages are stored in this item, allowing these messages to be easily logged with Logger or in the GOESQC data table. |
| $STATUS | x |  | x |  | text |  |
| $ERROR | x |  | x |  | text |  |
| station.$CHANNEL | x | x | x |  | number | Satellite channel number of last message decoded from a station. |
| station.$FAILURE_CODE | x | x | x |  | 71 = G - Good 63 = ? - Parity Errors 87 = W - Wrong Chan 68 = D - Dup Chan 65 = A - Addr Error 66 = B - Bad Address 84 = T - Time error 85 = U - Unexpected 77 = M - Missing Msg 73 = I - Invalid Address 78 = N - Incomp PDT 81 = Q - Bad Qual 67 = C - Compare Err 83 = S - Short Msg | ASCIIi numeric value of the failure code of the last message decoded from a station. |
| station.$SIGNAL_STRENGTH | x | x | x |  | 32-57 | Signal strength of the last message decoded from a station in dB, normally a message will be in the range 44 to 48. |
| station.$FREQUENCY_ERROR | x | x | x |  | -500 to +500 Hz | Frequency error of the last message decoded from a station in 50 Hz increments. +/- 250 Hz is the normal range. |
| station.$MODULATION | x | x | x |  | 49,60,71 degrees | Phase modulation of the last message decoded from a station. 60 degrees is normal, a value of 49 or 71 degrees may indicate a problem. |
| station.$DATA_QUALITY | x | x | x |  | 0 - Normal 1 - Fair 2 - Poor | Modulation quality of the last message decoded from a station. 0 is normal, 1 or 2 indicate that there may be some errors in the transmission. |
| station.$GOES_SPACECRAFT | x | x | x |  | E - GOES East W - GOES West C - GOES Central A - Argos | Indicates the location of the spacecraft which received the last message decoded from a station. |
| station.$ORIGINAL_LENGTH | x | x | x |  | number | Number of data bytes in the last message decoded from a station according to the length stored in the message. |
| station.$ACTUAL_LENGTH | x | x | x |  | number | Number of data bytes in the last message decoded from a station actually contained in the message. |
| station.$PARITY | x | x | x |  | number | Number of parity errors detected in the last message decoded from a station. |
| station.$SATID | x | x | x |  | number | Satellite ID of the last message decoded from a station. |
| DECODE filename |  |  |  | x | n/a | Executing the decode command will cause the passed filename to be decoded. The filename can include a path and/or wildcard characters. ex: DECODE C:\RAW\*.RAW would decode all non-decoded data in every file with an extension of "RAW" in the directory "C:\RAW". |
| STATUS |  |  | x |  | lines-of-text | Returns the contents of the XC Decode status display. |
| RROR |  |  | x |  | lines-of-text | Returns the contents of the XC Decode error display. |

Operations: A-Advise/UnAdvise, P-Poke, R-Request, E-Execute

### Setup Information

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [Decoder] | Run |  | List of programs to run on start up before the XC Decode starts to process data. ex: EXCEL.EXE |
|  | Clients |  | List of applications which must be running before files will be decoded. The names must match the window title of the application, which is usually similiar, but not the same as the executable file name or the DDE server name. ex: Microsoft Excel |
|  | AutoPath |  | Any files which match the AutoPath will checked for new data and/or decoded automatically based on the AutoInterval. The AutoPath may contain more that one directory to search. ex: C:\MESSAGES\*.RAW,C:\RAW\*.RAW Would automatically decode files ending in the extension ".RAW" contained in the directories MESSAGES and RAW on drive C:. |
|  | AutoInterval | 00:01:00 | Determines how often files specified by the AutoPath are check and/or processed. |

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [STATION:station] | Enable | No | Enables decoding of sensors from this station. |
|  | SiteType |  | Defines the type of station being decoded, so that smart decoding if available for the unit can be used. The following sitetypes have been defined: 8200 - Sutron 8200 Data Logger 8210 - Sutron 8210 RTU 9000 - Sutron 9000 RTU 9210 - Sutron 9210 (XLITE) RTU XPERT - Sutron XPERT RTU |
|  | SatID |  | 8 digit hexidecimal number representing the Satellite ID of the station. |
|  | Sensors |  | List of sensors used by this station and defined in the setup. |

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [SATID] | satid=station |  | [SATID] is a translation table used to translate Satellite ID's to station names. All satellite ID's to be decoded must be defined and have a matching station name. example: [SATID] 1234ABCD=HerndonCreek FA28478E=RestonDam |

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [CHANNEL] | channel=goestype | <=100 = GoesTimed >100 = GoesRandom | [CHANNEL] is a translation table used to determine from the channel a message was received on what type of decode processing should be used. For a GOES transmission a station usually needs to be decoded differently when the message is from a self timed channel versus a random alarm channel. Random alarm message are often much shorter. See the definition of [decodetype:station:sensor] to see how decoding a sensor is handled. Be sure to use 3 digits with leading zeros when defining the channel. example: [CHANNEL] 055=GoesTimed 167=GoesRandom 500=Argos |

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [decodetype:station:sensor] | Enable | No | Enables a sensor to be decoded when a message from station is received on a channel which requires decodetype processing. example: [GoesTimed:HerndonCreek:Stage] Enable=Yes |
|  | Format | 6BIT | Defines the type of data contained in the message. 6BIT and U6BIT are always used in GOES Random messages and often used in GOES Self timed messages. 8BIT and U8BIT are used in ARGOS messages. ASCII is used in GOES Self timed SHEF or Standard Decimal Format messages. 6BIT - Signed 6 Bit Binary U6BIT - Unsigned 6 Bit Binary 8BIT - Signed 8 Bit Binary U8BIT - Unsigned 8 Bit Binary ASCII - Signed or unsigned number |
|  | RelativeTime | No | Set this to Yes when this definition is a continuation of a decode and the time stamp where the previous decode table left off should be used. See the More option for information on how to continue a decode with another table. |
|  | RelativePos | No | Set this to Yes when this decode table is a continuation of a decode and you want the decode position to continue where the previous decode table left off. In some cases even with a continuation you will want to set this to NO in order to restart decoding at position 1. See the More option for information on how to continue a decode with another table. |
|  | AddBase | No | Setting this to Yes causes any decoded values to have a base value added to them. The base value is the value of the last decoded sensor in a previous decode table. This option allows you to decode messages where the first value is total count and the rest are just increments to that total count. See the More option for information on how to continue a decode with another table. |
|  | Size | 3 | Size is the length of the data to be decoded in bytes. For 6BIT data 3 bytes is the same as 18 bits. For 8BIT data 3 bytes is the same as 24 bits. |
|  | BitSize | 0 | BitSize is the length of the data to be decoded in bits. There is no need to use BitSize unless data is not encoded on byte boundaries. |
|  | Bound1 | 0 | Determines the position to start searching for a BlockID. |
|  | Bound2 | 0 | Determines the position not to seach past when looking for a BlockID in a message. |
|  | BlockID |  | This is a string which if specified is searched for in the message from Bound1 to Bound2 before decoding begins. If found the decode position is set to the character immediately after the BlockID. If the BlockID is left blank then no searching is performed and the initial decode position is set to the first data character in the message. |
|  | Pos | 1 | Determines the byte position of the sensor in the message. The first character at the current decode position is Pos 1. |
|  | BitPos | 1 | Determines the bit position of the sensor in the message. The most significant bit in a byte is at BitPos 1, the least significant bit in a byte is at position 8. BitPos is not needed unless the decode message is bit encoded. |
|  | Next | 3 | Determines how many bytes to increment the decode position to point to the next value of a sensor in a message. |
|  | BitNext | 0 | Determines how many bits to increment the decode position to point to the next value of a sensor in a message. BitNext is not needed unless the decode message is bit encoded. |
|  | Amount | 0 | Determines how many values for this sensor are contained in this message. |
|  | DeltaTime | 00:15:00 | This is the time betwen sensor values. If most recent data appears first in the message (which is the most common case) then DeltaTime should be negative. example: DeltaTime=-00:15:00 for typical 15 minute GOES data |
|  | RoundOffTime | 00:15:00 | Determines the interval at which time stamps are rounded off to. For instance with the default 15 minute value for RoundOffTime: 12:48 would be rounded to 12:45 and 12:53 would be rounded to 13:00. |
|  | OffsetTime | 00:00:00 | OffsetTime is the amount of time between when a DCP collects data and when it transmits it. Default time stamping of a sensor value is performed by subtracting the OffsetTime from the time the message was received and then rounding off to the nearest RoundOffTime interval. |
|  | More |  | Allows continuation of the decode of this sensor with a new XC Decode table by specifying the name of the table to use. You may need to use continuation decode table if the sensor values are irregular in some way. example: More=GoesTimed:HerndonCreek:MoreStage would continue decoding using the table information found under the topic [GoesTimed:HerndonCreek:MoreStage]. The name MoreStage is arbitrary and does not have to be defined as a sensor. In fact there is no requirement on the other names as well, except that the topic must contain decode table information. |
|  | SkipBlank | 0 | Will move the decode position past SkipBlank space characters after the block id in the message. This is useful when decoding standard decimal messages from the 8200/8210/9210/XPERT. An example of a typical sensor format of an 8200 ASCII message is: “:AT 0 #15 75.2 76.3” In order to skip past the offset of 0, and the interval of #15 to reach the first sensor value of 75.2, set SkipBlank to 3 - because there are 3 space characters. |
|  | SkipComma | 0 | Will move the decode position past SkipComma comma characters after the block id in the message. This is useful when decoding other vendor platform DCP messages. An example of a typical sensor format of an ASCII message is: “23.6,100.7,75.2,76.3” In order to skip past to read the third value, set SkipComma to 2. |
|  | SkipLF | 0 | Will move the decode position past SkipLF LF characters after the block id in the message. This is useful when decoding other vendor platform DCP messages. An example of a typical sensor format of an ASCII message is: “05.391 05.410 05.391 15778.6 15778.0 15772.4 14.34 14.11 14.21 " In order to read the values on third line, set SkipLF to 2. |

| TOPIC | ITEM | DEFAULT | DESCRIPTION |
| --- | --- | --- | --- |
| [SENSOR:station:sensor] | Enable | No | Set to Yes to allow the sensor to be decoded and processed. |
|  | TagName |  | to be used for automatic 8200/8210/9210/XPERT SHEF decoding. |
|  | Raw |  | If non-blank this is a list of additional sensors to process the raw data from this sensor through. Besides allowing the benefit of additional processing, Raw can also be used to store the same data in to multiple data files. ex: TempF,TempK would cause raw data to be processed by sensors named TempF, and TempK. |
|  | Equation |  | Allows an equation to be applied to incoming data to convert to engineering units or other processing.Table conversions and n-th order polynomials and trig functions are available. See the Equation field defined earlier in this document under the RTU Server program on page 32. An additional field called ASCII may be referenced. ASCII will be 1 when an ASCII message is being decoded, or 0 when a binary message is being decoded. This can be used as a flag to scale sensor values received in binary, but not in ASCII messages. ex: X/100+ 500 would move the decimal point of a sensor over two to the left value and add an offset of 500. ex: (ASCII=1)*(X*0.234+10.6)+(ASCII=0)*X converts the sensor with one equation when it is received in an ASCII message, and another when it is binary. Equations can also contain references to real time values: ex: (X+RTU01.AirTemp)/2 would average the air temperature with its most recent value. However, since GOES messages are usually decoded oldest data last, this would always result in averaging the current value with the most recent. |
|  | RightDigits | 0 | Determines the number of right decimal digits to retain when displaying or storing this number. |

## XC Decode COM Interface

### Introduction

The XC Decode is responsible for decoding sensor values from satellite RAW messags.  The decoded data when is stored in to a real time RAM resident data base.  The data values are then available to other programs such as Wonderware Intouch for display.  Setup information for Decode is stored in the XConnect.INI file and accessed via XC Setup.

XC Decode supports a variety of methods and events through its interface.  The XC Decode interface is named XCDecodeCOM.  The methods and events are described below. The methods and events are described below.  Most methods pass a group of parameters, a record, as a variant.  The order of the parameters in the table indicate is the order of the variant record that is passed.  Others functions will pass values as strings

| Type | Field Size |
| --- | --- |
| Boolean | 1 byte |
| Word | 2 bytes (unsigned) |
| Integer | 4 bytes (signed) |
| Longint | 4 bytes (signed) |
| Enumerated | 1 byte |
| Single | 4 bytes |
| Double | 8 bytes |

### XCDecodeCOM Methods

- DecodeFile

- StartDecode

- SetIgnorePos

- SetDecodeAll

- SetItemLink

- SetItemChange

- SingleItem

- ResetAllParams

- AppShutdown

- IsAlive

### XCDecodeCOM Events

- OnAppShutdown

- OnItemChange

- OnHeartBeat

### Methods

#### DecodeFile(FileName)

This procedures allows a client application to trigger XC Decode to decode a specific filename.  One parameter is required for this procedure: Filename.

#### StartDecode

This procedure allows a client application to trigger the decode cycle to check RAW files for new messages.

#### SetIgnorePos(Flag)

This procedure allows the user to control the Ignore Pos menu option.  When Ignore Pos is checked, XC Decoded will delete all the POS files and re-decoded all the satellite messages in ALL RAW files.  One parameter is required for this procedure:  True/False flag.

#### SetDecodeAll

This procedure allows the user to control the Decode Alls menu option.  Decode All is strictly a diagnostic tool.  All satellite messages in ALL RAW files will be decoded.  One parameter is required for this procedure:  True/False flag.

#### ResetlParams(AppGUIDStr)

This procedure allows the user to issues a software reset to all XConnect applications that retrieve information from XC Setup.  One parameter is required for the procedure call:  Application GUID string.  For the XConnect applications, when a ResetAllParams command is received, they will stop all processing, close any com ports and the re-request the latest setup information from XC Setup.

#### AppShutdown(AppGUIDStr)

This procedure will broadcast an application shutdown command to XConnect applications.  One parameter is required for the procedure call:  Application GUID string.  This method All XConnect servers and processing applications will respond to this command.  Applications that strictly connect to the database such as XC DataView, XC Reports or XC Export will not.

#### SetItemLink(AppGUIDStr, Item)

This procedure allows a client application to set a link to a specific data item.  XC Setup will send out an event to all clients via their GUID string with the new value.  Two parameters are required for this procedure call:  Application GUID string and item.  An item could be a specific sensor value such as XPERT.BATTERY.

#### SetItemChange(AppGUIDStr, Item, Data)

This procedure allows an application to broadcast an message for a data item value changed.  This provides data to any client applications with hot-links to data items.  Three parameters are required for the procedure call:  Application GUID string, Item, Data.

#### SingleItem(Item, Data)

This function allows the user to retrieve or set a single value from the internal real-time database.  Two parameters are required for the function call: Topic and Item.  The Topic is the parameter in the INI file surrounded by brackets such as [General].  The Item is the parameter entry in the topic, such as Use24Hour.  As a note, station, sensor and poll group information should not be deleted.  This information is stored in the database and will overwrite and user changes.

#### IsAlive(TimeString)

This procedure broadcasts a call with the current date time string (yyyy/mm/dd hh:mm:ss) to indicate the application is functioning properly and not frozen.

### Events

#### OnAppShutdown(AppGUIDStr)

This event notifies XConnect client applications when XC Setup will shutdown.  Many applications such as XC Rtu, XC Decode, XC Alarm or XC Calc require XC Setup, thus when XC Setup shuts down, they will also shutdown.

#### OnItemChange(AppGUIDStr, Item, Data)

This event notifies XConnect client applications when a data item has changed.  This provides data to any client applications with hot-links to data items.  Three parameters are passed with the event:  Application GUID string, Item, Data.

#### OnHeartBeat(AppGUIDStr, TimeString)

This event notifies XConnect applications of the application heartbeat.  Each application triggers a heartbeat.  XC Alarm monitors the heartbeat events.

# XC Alarm

## XC Alarm COM Interface

### Introduction

The XC Alarm is responsible for evaluating sensor thresholds and XConnect system alarms.  Setup information for the XC Alamr is stored in the XCAlarm.CFG file.  Sensor thresholds are stored in the database XC_SITESENSORS table.

XC Alarm supports a variety of methods and events through its interface.  The XC Alarm interface is named XCAlarmDataCOM.  The methods and events are described below. The methods and events are described below.  Most methods pass a group of parameters, a record, as a variant.  The order of the parameters in the table indicate is the order of the variant record that is passed.  Others functions will pass values as strings

| Type | Field Size |
| --- | --- |
| Boolean | 1 byte |
| Word | 2 bytes (unsigned) |
| Integer | 4 bytes (signed) |
| Longint | 4 bytes (signed) |
| Enumerated | 1 byte |
| Single | 4 bytes |
| Double | 8 bytes |

### XCAlarmDataCOM Methods

- REALTIME

- ALARM

- TIMETAG

- LASTUPDATE

- LASTALARM

- DBSTATUS

- AppShutdown

- SetItemLink

- SetItemChange

- SingleItem

- IsAlive

### XCAlarmDataCOM Events

- OnAppShutdown

- OnItemChange

### Methods

#### Realtime(Value)

This function allows the user to retrieve or set last data received from an Current Data or Alarm SSP message.  This returns the same data as $REALTIME DDE point.  The data return has the form of:

mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data, station.sensor,data...

#### Alarm(Value)

This function allows the user to retrieve or set last data received from an Alarm SSP message.  This returns the same data as $ALARM DDE point.  The data return has the form of:

mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data, station.sensor,data...

#### Timetag(Value)

This function allows the user to retrieve or set last data received from an Manual or Date data SSP message.  This returns the same data as $TIMETAG DDE point.  The data return has the form of:

mm/dd/yyyy hh:mm:ss,station.sensor,data, station.sensor,data, station.sensor,data...

#### LastUpdate(Value)

This function allows the user to retrieve or set last timestamp of data received from an Current Data or Alarm SSP message.  This returns the same data as $LASTUPDATE DDE point.  The data return has the form of:

mm/dd/yyyy hh:mm:ss

#### LastAlarm(Value)

This function allows the user to retrieve or set last alarm data evaluated by XC Alarm.  This returns the same data as $LASTALARM DDE point.  The data return has the form of:

07/22/2005 12:00:00,C,Test.Battery,H

#### DBStatus(Value)

This function allows the user to retrieve or set database status.  The data return has the form of: OK/FAIL.

#### AppShutdown(AppGUIDStr)

This procedure will broadcast an application shutdown command to XConnect applications.  One parameter is required for the procedure call:  Application GUID string.  This method All XConnect servers and processing applications will respond to this command.  Applications that strictly connect to the database such as XC DataView, XC Reports or XC Export will not.

#### SetItemLink(AppGUIDStr, Item)

This procedure allows a client application to set a link to a specific data item.  XC Setup will send out an event to all clients via their GUID string with the new value.  Two parameters are required for this procedure call:  Application GUID string and item.  An item could be a specific sensor value such as XPERT.BATTERY.

#### SetItemChange(AppGUIDStr, Item, Data)

This procedure allows an application to broadcast an message for a data item value changed.  This provides data to any client applications with hot-links to data items.  Three parameters are required for the procedure call:  Application GUID string, Item, Data.

#### SingleItem(Item, Data)

This function allows the user to retrieve or set a single value from the internal real-time database.  Two parameters are required for the function call: Topic and Item.  The Topic is the parameter in the INI file surrounded by brackets such as [General].  The Item is the parameter entry in the topic, such as Use24Hour.  As a note, station, sensor and poll group information should not be deleted.  This information is stored in the database and will overwrite and user changes.

#### IsAlive(TimeString)

This procedure broadcasts a call with the current date time string (yyyy/mm/dd hh:mm:ss) to indicate the application is functioning properly and not frozen.

### Events

#### OnAppShutdown(AppGUIDStr)

This event notifies XConnect client applications when XC Setup will shutdown.  Many applications such as XC Rtu, XC Decode, XC Alarm or XC Calc require XC Setup, thus when XC Setup shuts down, they will also shutdown.

#### OnItemChange(AppGUIDStr, Item, Data)

This event notifies XConnect client applications when a data item has changed.  This provides data to any client applications with hot-links to data items.  Three parameters are passed with the event:  Application GUID string, Item, Data.

# XC Calc

## XC Calc COM Interface

Oops, our apologies...topic still under construction.

# Templates

All topics can contain a Template item. A template contains the name of a new topic to search if an item cannot be found in the current topic. For instance if you are creating a station definition called STATION:NEW which is the same as STATION:OLD except for a few differences, you could set "STATION:NEW,Template" to the string "STATION:OLD". Then for instance if you requested "STATION:NEW,SiteType" and the site type did not exist in station NEW, then the XC Setup would look for it under "STATION:OLD,SiteType". Likewise if SiteType could not be found under station OLD then the XC Setup would examine "STATION:OLD,Template" if it was not blank and continue the search with that topic.

What the XCONNECT.INI file would look like:

[STATION:OLD]

SiteType=8200

ComPort=Com1

AckDelay=10.0

ResponeTimeout=30.0

UnitID=UNIT1

RadioPath=UNIT1

PhoneNumber=555-1212

Sensors=AirTemp,RainFall,WindSpeed,WindDir

[STATION:NEW]

; Default topics in station new to values contained in station old!

Template=STATION:OLD

; We only have to add information which is different:

UnitID=UNIT2

RadioPath=UNIT2

PhoneNumber=555-1213

Templates are very powerful because they allow you to concentrate like information in one place and reduce the amount of setup information in the setup file. Also in the case above station NEW did not have to be based on an existing "real" station, it could just as well be based on a fake topic which contains station general information. Template searching relies only on string substitution it does not rely on context information.

There are some limitations and special behaviour regarding template information. For one thing template linked items cannot be hot-linked, they can only be requested. Second template linked items can not be poked. If you want to change the base definition you will have to poke that item. If you wish to change the highest level value you will have to add a new item to that topic and then set it. This introduces an interesting difference between linked and non-linked items - that if you can request an item, but not change it then it must be linked.

Another special feature of the template handler in the XC Setup is the ability to associate a child topic with a father topic's template. For example, the sensor topic is a child of the topic station (since sensors belong to stations). When you tell the XC Setup to make this association it will ensure that any matching templates belonging to the father topic will also be inherited by the child. In simpler terms, if station NEW is based on station OLD and you do not explicitly define a sensor for station NEW then it will automatically be treated the same as the sensor defined in station OLD.

Below are the typical associations used for the RTU Server and other applications:

| TOPIC | ENTRY | DESCRIPTION |
| --- | --- | --- |
| [TEMPLATE] | ChildTopic=FatherTopic | This is the general format for associating any template of a child topic with the template of a father topic. |
|  | Sensor=Station | This causes unsuccessful requests for information under the topic [SENSOR:station:sensor] to look further using the sensor information contained in the station specified in the template contained in the topic [STATION:station]. |
|  | TagInfo=Station | This causes unsuccessful requests for information under the topic [TAGINFO:station] to look further using the tag information contained in the station specified in the template contained in the topic [STATION:station]. |
|  | LogInfo=Station | This causes unsuccessful requests for information under the topic [LOGINFO:station] to look further using the log information contained in the station specified in the template contained in the topic [STATION:station]. |
|  | GoesRandom=Station | This causes unsuccessful requests for information under the topic [GOESRANDOM:station:sensor] to look further using the GOES random information contained in the station specified in the template contained in the topic [STATION:station]. |
|  | GoesTimed=Station | This causes unsuccessful requests for information under the topic [GOESTIMED:station:sensor] to look further using the GOES timed information contained in the station specified in the template contained in the topic [STATION:station]. |

# NESDIS Header Field Description

CE12265402365235959G46-3NN052WFF00029@DEAap@JAAap@JAAap@JAgri

1       2 3  4 5 6 78 9 ABC  DE F    G                       H

| Field # | Byte Length | Range | Description |
| --- | --- | --- | --- |
| 1 | 8 | 0-9,A-F | Satellite ID, Platform ID, DCP Address |
| 2 | 2 | 00-99 | Year message was received relative to GMT |
| 3 | 3 | 000-365 | Julian day message was received relative to GMT |
| 4 | 2 | 00-23 | Hour message was received relative to GMT |
| 5 | 2 | 00-59 | Minute message was received relative to GMT |
| 6 | 2 | 00-59 | Second message was received relative to GMT |
| 7 | 1 | G,?,W,A,B,T,U, M,I,N,Q,C,S | Failure Code G - Good Message ? - Message received with parity errors M - Missing message S - Short or Truncated message (Sutron Only) THE FOLLOWING CODES ARE USED BY NESDIS ONLY: W - Message received on wrong channel D - Message received on multiple channel (duplicate) A - Message received with address error(s) (correctable) B - Message received with bad address (not correctable) T - Message received late/early (time error) U - Unexpected message I - Invalid address N - PDT entry for platform not complete Q - Bad quality measurements C - Comparison error on test transmission Note: The PDT is NESIDS's platform description table. |
| 8 | 2 | 32-57 | Signal Strength of received message in dB. Normal operation is 44-48. Reliable data can be received as low as 37 if not other signal problem exists. |
| 9 | 2 | 0,1,2...9,A -1,-2...-9,-A | Frequency Offset in 50 Hz increments. 0=0-49 Hz, 1=40-99 Hz,...8=400-449 Hz, A=500-549 Hz. A minus sign indicates the same range only below the center frequency. |
| A | 1 | N,H,L | Modulation Index or Phase Deviation N = Normal, 60 +/- 5 degrees H = High, > 70 degrees L = Low, < 50 degrees |
| B | 1 | N,H,P | Modulation Quality N = Normal, error rate better than 1x10-6 F = Fair, error rate between 1x10-4 and 1x10-6 P = Poor, error rate worse than 1x10-4 |
| C | 3 | 000-999 | Channel message was received on |
| D | 1 | E,W,C,A | Satellite Spacecraft Location E - GOES East W = GOES West C = GOES Central (not currently used) THE FOLLOW CODES HAVE BEEN DEFINED BY SUTRON TO SUPPORT NON-GOES SATELLITES: A = ARGOS (Never see this from NESDIS) |
| E | 2 | FF | Uplink carrier status (not used, always FF) |
| F | 5 | 00000-15750 | Message Data Length (plus 4 bytes to account for satellite ID which used to be contained in the message + 1 byte for the EOT stored as a blank space at the end). In the example, there are acutally only 24 bytes of data, however, 24+4+1 or 29 is the actual number stored in the data length. |
| G | Message Data Length minus 5 | !-~,CR,LF, SPACE | Message data. Can be continued on multiple lines separated by a CR and a LF. |
| H | 1 |  | Blank space representing EOT (End of Transmission) at end of message. |