XC Alarm

Sutron Corporation

21300 Ridgetop Circle

Sterling, Virginia 20166

TEL: (703) 406-2800

FAX: (703) 406-2801

WEB: http://www.sutron.com

Table Of Contents

Welcome to XC Alarm	1

Using XC Alarm	3

Moving around XC Alarm	3

The Menu Bar	3

The XC Alarm Toolbar	4

The following tools are provided:	4

Notification Panel	5

Alarms Status Panel	5

Editing email settings	6

Email settings parameters	6

Editing fax settings	8

Fax settings parameters	8

Editing wave file settings	10

Editing applications options	11

Application options parameters	11

Connecting to a remote servers	12

Messages in the status window	13

Logging alarm messages	13

Logging status messages	13

Reinitializing parameters	14

Advanced Topics	15

XConnect Database Schema	15

XConnect Database Schema	15

Troubleshooting	23

Error Messages	23

Error 1100: Cannot create Alarms message file.	23

Error 1101: Cannot write Alarms message to file.	23

Error 1104: Unable to connect to XC Calc Interface.	23

Error 1105: Unable to connect to XC Mux Interface.	23

Error 1106: Unable to connect to XC IMux Interface.	24

Error 1107: Unable to connect to XC Daps Interface.	24

Error 1108: Unable to send email. Error: message.	24

Index	25

Tables of Figures

Figure 1.  XC Alarm main window	3

Figure 2.  .  Email settings window	6

Figure 3.  Fax settings window	8

Figure 4.  Wave file settings window	10

Figure 5.  Application options window	11

Figure 6.  Remote Connect window	12

# Welcome to XC Alarm

XConnect is Sutron Corporation's latest data collection, data processing and data storage software.  Built on the strong principles of PcBase2, XConnect is compliant with today's 32-bit Windows operating systems and provides new tools and options for the user.

XConnect is a collection of executables designed to provide a complete solution for data collection, data handling, data viewing and data storage.

XC Alarms's role in an XConnect system helps the user evaluate sensor thresholds and alarms, if desired.  Additionally, when alarm thresholds are exceeded the user can configure notifications by fax, email or text messages.

XC Alarm also allows the user to monitor XConnect applications and data.  Hearbeat checks will be made to XConnect applications.  Also, database connectivity will also be monitored every minute.  When database connectivity is restored, a ReInitialize will be sent to all the servers to re-connect to the database.

For real-time systems such as flood warning systems, XC Alarm can serve to immediately notify users of high or rate-of-change thresholds.  For all systems, XC Alarm can monitor XConnect applications are all running and responding as well as for incoming data.

XC Alarm is used in conjunction with the database data storage option and the XConnect database schema.  The user thresholds and alarm limits are entered in XC Desktop in the Sensor Setup window.  All alarm thresholds are stored in the XC_SITESENSORS data table.

An example of the format of the alarm message sent is:

[2004/07/08 13:28:03] HighAlarm for Citizens.Stage @ 2004/07/07 12:15:00 : 3.08 above limit 1.2

# Using XC Alarm

## Moving around XC Alarm

The XC Alarm application consists of these main areas:

- The Menu Bar provides access to all user-controllable functions within the application.

- The Toolbar provides one-click access to important functions.

- The Alarms Display windows displays the real-time data.

- The Status panel provides the user with informational or error messages as well as queues for alarm faxes and alarm emails.

Figure 1.  XC Alarm main window

## The Menu Bar

The Menu Bar provides access to the following menus:

| File | - ReInitialize -- reloads the latest setup information from XC Setup and re-connects to local XC Rtu and/or XC Decode. - Login -- user login for XConnect system. - Exit -- terminate the application. |
| --- | --- |
| Alarms | - Ack All Alarms -- acknowledges all alarms on the Alarms memo box. - Ack Selected Alarms -- acknowledges selected alarms on the Alarms memo box. |
| Options | - Remote Nodes -- connect to remote XConnect servers as source of data. - Log Alarms -- logs all messages in the Alarms message windows to a disk file. - Log Status Messages -- logs all status messages to a disk file. - Wave File Settings -- edit wave file names used for audible alarms. - Email Settings -- edit settings for email alarm notification. - Fax Settings -- edit settings for fax alarm notification. - Application Settings -- edit setup parameters for XC Alarm. - Clear Status Messages -- clear the status message window. - Clear Alarm Messages -- clear the messages in the Alarms message window. - Clear Acked Alarm Messages -- clear the messages in the Acked Alarms message window. |
| Help | - XC Alarm Help -- access this help system an defaults to Index tab. - XC Alarm Contents -- access this help system an defaults to Contents - About -- get version information for this application. |

## The XC Alarm Toolbar

The toolbar area allows you to quickly access various XC Alarm functions.

### The following tools are provided:

|  | Reload the latest setup information from XC Setup and re-connects to local XC Rtu and/or XC Decode. |
| --- | --- |
|  | Acknowledge all alarms on the Alarms memo box. |
|  | Acknowledge selected alarms on the Alarms memo box. |
|  | Log all messages in the Alarms message windows (new alarms and 'acked' alarms) to a disk file. |
|  | Log all status messages to a disk file. |
|  | Connect to remote XConnect servers. |
|  | Edit wave file names used for audible alarms. |
|  | Edit settings for email alarm notification. |
|  | Edit settings for fax alarm notification. |
|  | Edit setup parameters for XC Alarm. |
|  | Open the on-line help (this document). |

## Notification Panel

The panel is a multi-tabbed control that displays status/error messages from the XC Alarm application as well as the current queues for any alarm notifications dispatched via fax or email.

## Alarms Status Panel

The Alarms status windows updates in real-time all the alarms that have been received and acknowledged.  All new alarms are displayed in the top status window.  Whenever the user acknowledges any alarms, they will be removed from the top status window to the lower Acked Alarms status window.

## Editing email settings

XC Alarm has a few settings required for sending e-mail alarm notifications.

- Click on XC Alarm to start the application

- From the Options menu, select Application Settings or from the toolbar click the    button.

- The Email Settings window appears.

Figure 2.  .  Email settings window

### Email settings parameters

- Mail Type - XC Alarm currently only support Simple Mail Transfer Protocol (SMTP).  SMTP is a TCP/IP protocol used in sending and receiving e-mail.  SMTP usually is implemented to operate over Internet port 25.  Please confirm this your network/IT personnel.

- Enable - Check this option to enable e-mail alarm notifications.

- SMTP Host - Enter mail server hostname or IP address (i.e, mymail.server.com or 192.234.22.12).

- From - Enter senders email address (i.e., jdoe@mycompany.com).

- Port  - Enter Internet port number used for SMTP.  Typically this will be set to 25.

- To - Enter recipients email address.  For multiple recipients separate with semicolons (i.e., adoe@mycompany.com;bdoe@mycompany.com;jdoe@mycompany.com).

- Cc - Enter additional email address to copy.  To enter multiple address, separate address with semicolons.

- Authentication - SMTP authentication is an extension to the SMTP protocol that allows you to prove your identity to a mail gateway.  SMTP Authentication allows for roaming access and reduces the SPAM that increases the traffic overhead on our mail servers by providing verification that mail originates from a valid user. Verify with your IT personnel if login authentication is needed on the XConnect PC running XC Alarm.

- Username - Username for SMTP authentication when sending email.

- Pass - Password for SMTP authentication when sending email.

## Editing fax settings

XC Alarm has a few settings required for sending fax alarm notifications.

- Click on XC Alarm to start the application

- From the Options menu, select Application Settings or from the toolbar click the    button.

- The Fax Options window appears.

Figure 3.  Fax settings window

### Fax settings parameters

- Fax Class - Indicates whether the fax modem is used as Class 1, Class 1_0, Class 2 or Class 2.0.  Refer to you fax modem user's manual.  Selecting auto detect is a good default choice.

- Enable - Check this option to enable fax alarm notifications.

- Modem - Select fax modem used to fax alarm notifications.

- Phone Number - Specifies the number to dial.

- Dial attempts - Determines then number of times the phone number will be dialed to establish a fax session.

- Retry wait - Number of seconds to wait after a busy signal before trying the number again.

- Fax header - Title of the fax.  $F represents the Sender ID, $D represents the date as mm/dd/yyyy and $T represents the time as HH:MM pm.  if $F, $D, or $T are used, the current date, time and user entered Sender ID will be substituted in the fax header.

- Sender ID – Sender name/identification.

- Dial Prefix - The Dial Prefix specifies an optional dial prefix that is inserted in the dial command between the "ATDT" and the number to dial.  If your telephone system requires special characters or codes when dialing out, you can specify then once here rather than in every fax number.  Do not include the "ATD" or a "T" or "P" tone/pulse modifier in the dial prefix.  the "ATD" is automatically prefix.

- Modem Init - A custom modem initialization string.  Do not include the "AT" in the string.

- Fax Header -  Custom header for the fax.  Key codes $F, $D, $T will automatically include the Sender ID, Date and Time, respectively.

- Use Enhanced Text for Fax Headers - Determines if user-selected fonts are used in the fax.  The default font for the header is Arial 11pt.

- Header Font - User-selected font for the header text.

## Editing wave file settings

XC Alarm supports audible warnings when thresholds are exceeded.  Users can specify a different wave file for each alarm condition.  Alarm wave files can be easily recorded using Windows Sound Recorder application.

- Click on XC Alarm to start the application

- From the Options menu, select Wave File Settings or from the toolbar click the    button.

- The Alarm wave file window appears.

Figure 4.  Wave file settings window

The user can select which alarms they would like evaluated and then select the corresponding wave file to be played when then alarm condition occurs.  If multiple alarms occur, the highest level alarm wave file will be played.  The alarm wave file will play continuously until the user closes the

## Editing applications options

XC Alarm has a few options apart from those for email and fax.

- Click on XC Alarm to start the application.

- From the Options menu, select Application Settings or from the toolbar click the    button.

- The Applications Options window appears.

Figure 5.  Application options window

### Application options parameters

- Notification Settings - XC Alarm can be instructed to not exceed a limit total emails or faxes that will be transmitted within a certain time period.  This will prevent a flood of repeated emails or faxes that may be triggered if a sudden system flood/alarm event occurs.

- Monitoring Settings - XC Alarm can monitor the heartbeat of XConnect applications. This ensures that all the applications are running and responding.  XC Alarm checks the heartbeat of XConnect applications every minute.  If, after the user-specified time, the heartbeat response is not received by an application, an alarm notifications will be sent.

- Data Settings - XC Alarm can monitor data continually.  If, after the user-specified time, the new data has not been received by XC Alarm, an alarm notifications will be sent.

  - No data received by system - XC Alarm can send alarm if the system has not received data from ANY station in the user-selected time.

  - No data received by station - XC Alarm can send alarm if the data has not been received data from EACH station in the user-selected time.

  - Evaluate System Alarms only - XC Alarm can configured to only evaluate the system alarms and not sensor alarms, even if sensor threshold settings have been entered.

- Status Detail  - User can select the level of status messages displayed in the status window.

## Connecting to a remote servers

Normally, XC Alarm will connect to the XConnect servers running on the same computer. Since XConnect modules are automation servers, connections can be made remotely over a network.  In many cases, the XConnect base station computer that is controlling communications is in a separate office or building than the users. In this situation, the users can run XC Alarm on his/her local machine and connect over the network to the XConnect servers on the XConnect base station. The data collected on the remote computer node can be displayed by the XC Alarm running on the local machine.

To connect to a remote XConnect servers:

- From the main menu, select Options|Remote Nodes or click on the  to select a computer node on the network.

- After selecting the XConnect base station computer node, XC Alarm will display all real-time data received by the XC Rtu or XC Decode on the remote mode.

Figure 6.  Remote Connect window

## Messages in the status window

Below are the status messages XC Alarm will display in the status window as it evaluates alarm conditions:

- Sending email.

Email jobs are commencing.

- Connecting email server hostname:port number.

Attempting to connect to SMTP host server at designated port number.

- Email server connected.

Connection to email server successful.

- Email sent successfully.

Email was successful.

- Transmit finished successfully. Seq = num; Priority = num Fax = xxx; Scheduled Time = time.

Fax was transmitted successfully.

- Transmit started. Seq = num; Priority = num Fax = xxx; Scheduled Time = time.

Fax transmission has been initiated.

- Transmit failed. Seq = num; Priority = num Fax = xxx; Scheduled Time = time.

Fax transmission failed.

- Fax Receiving is not supported.

There was an attempt to send a fax to this location.  Fax receiving is not supported in XC Alarm.

## Logging alarm messages

All alarms new and acknowledged can be logged to disk. To log all status  messages:

- Click the XC Alarm icon on the desktop. The XC Alarm application starts minimized.

- From the Options menu, select Log Alarms or from the toolbar click on the  icon.  The user will be required to enter/select the name of the file to store the new alarm messages.

- Messages will be stored in an subdirectory named Alarms of the XConnect installation directory (i.e., C:\XCONNECT\ALARMS).

## Logging status messages

The purpose of the Log Status Messages is more for troubleshooting.  Logging all messages, it will provide Sutron customer support as a historical log of all events to help troubleshoot any problems.  To log all status  messages:

- Click the XC Alarm icon on the desktop. The XC Decode application starts minimized.

- From the Options menu, select Log Status Messages or from the toolbar click on the  icon.  The user will be required to enter/select the name of the file to store the status messages.

## Reinitializing parameters

XC Alarm gets the configuration data it needs from the XC Setup program.  Since most of this information is requested at application startup, there needs to be a mechanism for propagating changes that are made to the setup after XC Alarm is already running.  This mechanism is provided via the Reinitialize function which can be found in XC Alarm on the toolbar or under the FILE menu.  Clicking the Reinitialize button causes XC Alarm to flush the current setup information from its buffer,  request new setup information from XC Setup and most importantly, re-connects to the XC Rtu or XC Decode.

# Advanced Topics

## XConnect Database Schema

### XConnect Database Schema

XConnect consists of a set of 4 “management” tables and a set of 4 "data" tables.  The XConnect tables are designed to provide a great deal of flexibility in:

- Naming of stations and data parameters

- Grouping data through such attributes as river basin or political boundaries

- Managing the size of data tables so that performance is maximized

The four management tables are:

- XC_SITES - Contains stations in XConnect system.

- XC_SITESENSORS - Contains sensors of stations in XConnect system.

The four data tables are:

- XC_DATA1 - Contains numerical sensor data.

- XC_TXTDATA1 - Contains text sensor data.

- XC_PPDATA1 - Contains data from post-processing instructions and operations.

- XC_GOESQC - Contains satellite message quality data.

- XC_RTUQC - Contains SSP communication data.

#### XC_SITES

| Name | Data Type | Length | Required | Description |
| --- | --- | --- | --- | --- |
| STATION_ID (PK) | Character | 20 | Y | Name of station in XConnect |
| ENABLED | Character | 1 | Y | Active station (Y/N) |
| UNIT_ID | Character | 15 | N | RTU Unit ID |
| SITE_COMMENT | Character | 32 | N | General purpose string for comments on station |
| INSTALLATION_DATE | Date |  | N | Date station was installed |
| AGENCY | Character | 32 | N | Agency or organization that owns the station |
| COUNTRY | Character | 32 | N | Country where site is located |
| DISTRICT | Character | 32 | N | District where station is located |
| CITY | Character | 32 | N | City where station is located |
| STATE | Character | 32 | N | State where station is located |
| COUNTY | Character | 32 | N | County where station is located |
| BASIN | Character | 32 | N | Rriver basin where station is located |
| LATITUDE | Number |  | N | Latitude of station location |
| LONGITUDE | Number |  | N | Longitude of station location |
| ELEVATION | Number |  | N | Elevation of station |
| WEIGHTING_FACTOR | Number |  | N | Precipation station weighting for map calculation |
| COEFFICIENT1 | Number |  | N | General purpose coefficient 1 |
| COEFFICIENT2 | Number |  | N | General purpose coefficient 2 |
| COEFFICIENT3 | Number |  | N | General purpose coefficient 3 |
| SATELLITE_ID | Character | 8 | N | Satellite/NESDIS identifier |
| PRIMARY_CHANNEL | Number |  | N | Primary satellite transmission channel |
| RANDOM_CHANNEL | Number |  | N | Secondary satellite transmission channel |
| REPORTING_TIME | Time |  | N | Assigned satellite transmission time slot |
| REPORTING_INTERVAL | Number |  | N | Assigned satellite transmission interval |
| LAST_UPDATE | Date |  | N | Time of last data received for station |
| TZONE_CODE | Character | 3 | N | ASCII time zone code, e.g., EST |
| GMT_OFFSET | Number |  | N | Time zone offset from GMT |
| SHEF_ID | Character | 8 | N | SHEF code for station |
| ALTERNATE_CHAR_ID_1 | Character | 10 | N | Alternate character ID 1 |
| ALTERNATE_CHAR_ID_2 | Character | 10 | N | Alternate character ID 2 |
| GPSTRING1 | Character | 20 | N | General purpose string 1 |
| GPSTRING2 | Character | 20 | N | General purpose string 2 |
| GPNUMBER1 | Number |  | N | General purpose number 1 |
| GPNUMBER2 | Number |  | N | General purpose number 2 |
| SETUP_FILE_NAME | Character | 255 | N | Filename of data logger setup file |
| PICTURE_FILE_NAME | Character | 255 | N | Filename of image of station |

#### XC_SITESENSORS

| Name | Data Type | Length | Required | Description |
| --- | --- | --- | --- | --- |
| STE_STATION_ID (FK) | Character | 20 | Y | Name of station in XConnect |
| SENSORNAME (PK) | Character | 12 | Y | Name of sensor in XConnect |
| ENABLED | Character | 1 | Y | Active sensor enabled for porcessing (Y/N) |
| DATA_TABLE | Character | 10 | Y | Data table name where sensor is stored |
| DESCRIPTION | Character | 32 | N | General purpose string for comments on sensor |
| MANUFACTURER | Character | 20 | N | Manufacturer of sensor |
| MODEL_NUMBER | Character | 20 | N | Model number of sensor |
| SHEF_PE_CODE | Character | 7 | N | SHEF code for sensor |
| PARAMETER_CODE | Character | 12 | N | Parameter code applied for sensor |
| ALT_PARAMETER_CODE_1 | Character | 12 | N | Alternate parameter code 1 |
| ALT_PARAMETER_CODE_2 | Character | 12 | N | Alternate parameter code 2 |
| EQUATION | Character | 128 | N | Equation applied to sensor |
| RATING_TABLE | Character | 128 | N | Filename of rating/lookup table applied to sensor |
| OFFSET | Number |  | N | General purpose offset number |
| SHIFT | Number |  | N | General purpose shift number |
| HIGH_HIGH_LIMIT | Number |  | N | High high limit for sensor |
| HIGH_HIGH_ALARM | Number |  | N | How to apply High High limit: NONE, ABOVE, BELOW |
| HIGH_LIMIT | Number |  | N | High limit for sensor |
| HIGH_ALARM | Number |  | N | How to apply High limit: NONE, ABOVE, BELOW |
| LOW_LIMIT | Number |  | N | Low limit for sensor |
| LOW_ALARM | Number |  | N | How to apply Low limit: NONE, ABOVE, BELOW |
| LOW_LOW_LIMIT | Number |  | N | Low low limit for sensor |
| LOW_LOW_ALARM | Number |  | N | How to apply Low Low limit: NONE, ABOVE, BELOW |
| ROC_LIMIT | Number |  | N | Rate of change limit for sensor (per second) |
| ROC_ALARM | Number |  | N | How to apply Rate of Change limit: NONE, ABOVE, BELOW |
| ROC_INTERVAL | Number |  | N | Interval (seconds) over which rate of change is computed |
| DEADBAND | Number |  | N | Tolerance or allowable change in No Change where the alarm will not trigger |
| NO_CHANGE_INTERVAL | Number |  | N | Maximum time allowed for no change in sensor value expressed in days and fractions |
| NO_CHANGE_ALARM | Number |  | N | How to apply No Change limit: NONE, ON |
| TEXT_LIMIT | Character | 32 | N | Text that corresponds to an alarm condition for a text sensor data |
| TEXT_ALARM | Number |  | N | How to apply Text alarm: NONE, ON |
| COEFFICIENT1 | Number |  | N | General purpose coefficient 1 |
| COEFFICIENT2 | Number |  | N | General purpose coefficient 2 |
| COEFFICIENT3 | Number |  | N | General purpose coefficient 3 |
| COEFFICIENT4 | Number |  | N | General purpose coefficient 4 |
| COEFFICIENT5 | Number |  | N | General purpose coefficient 5 |
| GPNUMBER1 | Number |  | N | General purpose number 1 |
| GPNUMBER2 | Number |  | N | General purpose number 2 |
| GPSTRING1 | Character | 20 | N | General purpose string 1 |
| GPSTRING2 | Character | 20 | N | General purpose string 2 |
| ALERT_ID | Number |  | N | ALERT identifier for sensor |
| ALERT_TYPE | Character | 10 | n | ALERT sensor type |

#### XC_DATA1

| Name | Data Type | Length | Required | Description |
| --- | --- | --- | --- | --- |
| STATION_ID (PK) | Character | 20 | Y | Name of station where sensor is located |
| SENSORNAME (PK) | Character | 12 | Y | Name of sensor |
| TIME_TAG (PK) | Date |  | Y | Date and time sensor value received |
| SOURCE (PK) | Character | 1 | Y | Source of data. Valid options are: S (Self-timed), R (Random), C (Current), T (Timetag) |
| ORIG_VALUE | Number |  | Y | Original data value as processed by XC Rtu or XC Decode |
| ED_VALUE | Number |  | N | Edited data value |
| FLAG1 | Character | 1 | N | General purpose quality flag 1 |
| FLAG2 | Character | 1 | N | General purpose quality flag 2 |
| FLAG3 | Character | 1 | N | General purpose quality flag 3 |
| FLAG4 | Character | 1 | N | General purpose quality flag 4 |
| HIGH_HIGH_FLAG | Number |  | N | Status of high high flag. Valid options are: 0 (NORMAL), 1 (ABOVE), -1 (BELOW) |
| HIGH_FLAG | Number |  | N | Status of high flag. Valid options are: 0 (NORMAL), 1 (ABOVE), -1 (BELOW) |
| LOW_FLAG | Number |  | N | Status of low flag. Valid options are: 0 (NORMAL), 1 (ABOVE), -1 (BELOW) |
| LOW_LOW_FLAG | Number |  | N | Status of low low flag. Valid options are: 0 (NORMAL), 1 (ABOVE), -1 (BELOW) |
| ROC_FLAG | Number |  | N | Status of rate of change flag. Valid options are: 0 (NORMAL), 1 (ABOVE), -1 (BELOW) |
| NO_CHG_FLAG | Number |  | N | Status of no change flag. Valid options are: 0 (NORMAL), 1 (CHANGE) |
| ALARM_FLAG | Number |  | N | Status of alert flag. Valid options are: 0 (NORMAL), 1 (CHANGE) |

#### XC_TXTDATA1

| Name | Data Type | Length | Required | Description |
| --- | --- | --- | --- | --- |
| STATION_ID (PK) | Character | 20 | Y | Name of station where sensor is located |
| SENSORNAME (PK) | Character | 12 | Y | Name of sensor |
| TIME_TAG (PK) | Number |  | Y | Date and time senor value received |
| SOURCE (PK) | Character | 1 | Y | Source of data. Valid options are: S (Self-timed), R (Random), C (Current), T (Timetag) |
| TEXT_VALUE | Character | 256 | Y | Original text value as received by XC Rtu or XC Decode |
| FLAG1 | Character | 1 | N | General purpose quality flag 1 |
| FLAG2 | Character | 1 | N | General purpose quality flag 2 |
| FLAG3 | Character | 1 | N | General purpose quality flag 3 |
| FLAG4 | Character | 1 | N | General purpose quality flag 4 |
| MESSAGE_STATUS | Character | 2 | N | Status character used to track if message has been read, forward, etc. |
| TXT_ALARM_FLAG | Number |  | N | Status of text alarm flag. Valid options are: 0 (NORMAL), 1 (ALARM) |

#### XC_PPDATA1

| Name | Data Type | Length | Required | Description |
| --- | --- | --- | --- | --- |
| STATION_ID (PK) | Character | 20 | Y | Name of station where sensor is located |
| DATA_LABEL (PK) | Character | 20 | Y | Name of computed value |
| PPI_NAME (PK) | Character | 5 | Y | Name of post processing instruction |
| TIME_TAG (PK) | Date |  | Y | Date and time of start of the post processing time period |
| TIME_PERIOD (PK) | Number |  | Y | Number of hours/days of data used by function |
| PP_VALUE | Number |  | Y | Post processed value |
| FLAG1 | Character | 1 | N | General purpose quality flag 1 |
| FLAG2 | Character | 1 | N | General purpose quality flag 2 |
| FLAG3 | Character | 1 | N | General purpose quality flag 3 |
| VALUE_TIME_TAG | Date |  | N | Exact time value occurred (i.e, Time of max value) |
| PROCESS_TIME_TAG | Date |  | Y | Date and time value was actually processed/calculated |
| SENSORNAME | Character | 12 | Y | Name of sensor calculations performed on |
| ALTERNATE_LABEL | Character | 20 | N | Alternate label for computed value |

#### XC_GOESQC

| Name | Data Type | Length | Required | Description |
| --- | --- | --- | --- | --- |
| NESDIS_ID (PK) | Character | 8 | Y | NESDIS or satellite agency identifier |
| TIME_TAG (PK) | Date |  | Y | Date and time message was transmitted |
| SOURCE (PK) | Character | 1 | Y | Source of message. Valid options are: S (Self-timed), R (Random) |
| STATION_ID | Character | 20 | N | XConnect station name, if not defined, defaults to NESDIS ID |
| FAILURE_CODE | Character | 1 | N | Message status |
| SIG_STRENGTH | Number |  | N | Signal Strength |
| FREQ_OFFSET | Number |  | N | Frequency Offset |
| MOD_INDEX | Character | 1 | N | Modulation Index |
| MOD_QUAL | Character | 1 | N | Modulation Quality |
| CHANNEL | Number |  | N | Channel message transmitted on |
| SAT_LOC | Character | 1 | N | GOES satellite message transmitted to |
| UC_STATUS | Character | 2 | N | Uplink carrier status (NOT CURRENTLY USED - ALWAYS FF) |
| MESSAGE_LEN | Number |  | N | Transmitted message length |
| ACTUAL_LEN | Number |  | N | Actual received message length |
| PARITY NUMBER | Number |  | N | Number of parity characters in message |
| BUFFER_EMPTY | Number |  | N | Number of BUFFER EMPTY messages transmitted by station (SATLINK only) |
| BATTERY_VOLTAGE | Number |  | N | Battery voltage at station when message was transmitted |
| SPARE1 | Number |  | N | Spare field for future use |
| SPARE2 | Number |  | N | Spare field for future use |

#### XC_RTUQC

| Name | Data Type | Length | Required | Description |
| --- | --- | --- | --- | --- |
| STATION_ID (PK) | Character | 20 | Y | Name of the station SSP message was transmitted/received to/from |
| TIME_TAG (PK) | Date |  | Y | Date and time SSP message value transmitted/received |
| RX_TX (PK) | Character | 1 | Y | SSP messages was transmitted or received by XConnect |
| PORT | Character | 30 | N | Port SSP message was sent/received (i.e., COM!, COM2....) |
| SEQUENCENUM | Number |  | N | Sequence/flag number used in SSP message. |
| OPCODE | Number |  | N | SSP operation code (CurDataReq, TimeTagReq) |
| STATUS | Number |  | N | Status of SSP message (0 - None, 1 - Success, 2 - Failed) |
| RETRIES | Number |  | N | Number of retries used |
| GOOD | Number |  | N | Number of good messages received by the station |
| BAD | Number |  | N | Number of bad messages received by the station |
| CRC | Number |  | N | Number of SSP messages with CRC errors received by the station |
| BATTERY_VOLTAGE | Number |  | N | Battery Voltage from SSP message |
| SPARE1 | Number |  | N | Spare field for future use |
| SPARE2 | Number |  | N | Spare field for future use |

# Troubleshooting

## Error Messages

Potential error messages generated by XC Alarm are:

### Error 1100: Cannot create Alarms message file.

#### Troubleshooting steps:

Ensure that:

- File is not already in use by another application such as Notepad.

- Ensure directory path of output file exists.  Use Windows Explorer or My Computer to verify path.

### Error 1101: Cannot write Alarms message to file.

#### Troubleshooting steps:

Ensure that:

- Directory is not write-protected.

Go to Windows Explorer and verify the properties of the directory.

- File is not already open by another application.

### Error 1104: Unable to connect to XC Calc Interface.

#### Troubleshooting steps:

A COM interface connection is made by the XC Alarm application to XC Calc.  This allows XC Alarm to monitor events triggered by XC Calc when new sensor data is available.

Potential reasons for error:

- XC Calc is not running.

Close current application and restart.  XC Calc should start automatically.

- XC Calc does not start automatically with XConnect application.

Close current application.  Start XC Calc manually from Start Menu.  Then re-start XConnect applications.

If error still persists, reboot computer.

### Error 1105: Unable to connect to XC Mux Interface.

#### Troubleshooting steps:

A COM interface connection is made by the XC Alarm application to XC Mux.  This allows XC Alarm to monitor the heartbeat of XC Mux and ensure it is running properly.

Potential reasons for error:

- XC Mux is not running.

Close current application and restart.  XC Mux should start automatically.

- XC Mux does not start automatically with XConnect application.

Close current application.  Start XC Mux manually from Start Menu.  Then re-start XConnect applications.

If error still persists, reboot computer.

### Error 1106: Unable to connect to XC IMux Interface.

#### Troubleshooting steps:

A COM interface connection is made by the XC Alarm application to XC INSAT Mux.  This allows XC Alarm to monitor the heartbeat of XC INSAT Mux and ensure it is running properly.

Potential reasons for error:

- XC INSAT Mux is not running.

Close current application and restart.  XC INSAT Mux should start automatically.

- XC INSAT Mux does not start automatically with XConnect application.

Close current application.  Start XC INSAT Mux manually from Start Menu.  Then re-start XConnect applications.

If error still persists, reboot computer.

### Error 1107: Unable to connect to XC Daps Interface.

#### Troubleshooting steps:

A COM interface connection is made by the XC Alarm application to XC Daps.  This allows XC Alarm to monitor the heartbeat of XC DAPS and ensure it is running properly.

Potential reasons for error:

- XC Daps is not running.

Close current application and restart.  XC Daps should start automatically.

- XC Daps does not start automatically with XConnect application.

Close current application.  Start XC Daps manually from Start Menu.  Then re-start XConnect applications.

If error still persists, reboot computer.

### Error 1108: Unable to send email. Error: message.

#### Troubleshooting steps:

Ensure that:

- SMTP mail host name is correct.

- SMTP port number is correct.

- SMTP mail host may or may not require authentication.

# Index

A

Ack All Alarms	3

Ack Selected Alarms	3

Acked Alarms	3

ALARM	15

ALARM_FLAG	15

Alarms Status Panel	5

Application Settings	11

applications options

Editing	11

applications options	11

Applications Options window	11

C

Clear Acked Alarm Messages	3

Clear Alarm Messages	3

Clear Status Messages	3

Connecting

remote servers	12

Connecting	12

E

Editing

applications options	11

email settings	6

wave file settings	10

Editing	6

Editing	10

Editing	11

e-mail

receiving	6

sending	6

e-mail	6

Email

sending	6

Email	6

Email Settings	3, 6

Error Messages	23

F

Fax Settings	3

FLAG1	15

FLAG2	15

FLAG3	15

FLAG4	15

H

HIGH_ALARM	15

HIGH_FLAG	15

HIGH_HIGH_ALARM	15

HIGH_HIGH_FLAG	15

HIGH_HIGH_LIMIT	15

HIGH_LIMIT	15

L

Log Alarms	3, 13

Log Status Messages	3, 13

Logging

alarm messages	13

status messages	13

Logging	13

Logging	13

LOW_ALARM	15

LOW_FLAG	15

LOW_LIMIT	15

LOW_LOW_ALARM	15

LOW_LOW_FLAG	15

M

Mail Type	6

Menu Bar	3

Modem - Select	8

Modem Init	8

Monitoring

Settings	11

Monitoring	11

Moving

around XC Alarm	3

Moving	3

N

NO_CHANGE_ALARM	15

NO_CHANGE_INTERVAL	15

Notification Panel	5

Notification Settings	11

O

Options menu	6, 8, 10, 11, 13

Options|Remote Nodes	12

P

parameters

Reinitializing	14

parameters	14

Phone Number	8

Port - Enter Internet	6

R

ReInitialize	3

Reinitialize button	14

Reinitializing

parameters	14

Reinitializing	14

Remote Nodes	3

remote servers

Connecting	12

remote servers	12

ROC_ALARM	15

ROC_FLAG	15

ROC_INTERVAL	15

ROC_LIMIT	15

S

Sender ID	8

sending

email	6

sending	6

Simple Mail Transfer Protocol	6

SMTP Authentication	6

SMTP Host	6

Status	3

status messages

Logging	13

status messages	13

status/error	5

T

TCP/IP	6

TEXT_ALARM	15

TEXT_LIMIT	15

TEXT_VALUE	15

Troubleshooting

steps	23

Troubleshooting	23

U

Use Enhanced Text	8

Username

SMTP	6

Username	6

W

Wave File Settings

Editing	10

Wave File Settings	3

Wave File Settings	10

Welcome

XC Alarm	1

Welcome	1

Windows Sound Recorder application	10

X

XC Alarm application	3

XC Alarm Toolbar	4

XC_SITESENSORS	15

XConnect Database Schema	15